ECEN 4/5632         Theory and Application of Digital Filters          Fall 2026

10-26-26                                                                                                                                           P. Mathys

Quiz 5

1. Convolution

An N-point DFT multiplication of two sequences implements what kind of convolution by default?

2. Aliasing

A length-50 sequence is convolved with a length-17 FIR by FFT. What minimum transform length avoids time aliasing?

3. OLA vs OLS

In one sentence, distinguish overlap-add from overlap-save.

4. STFT duration

An STFT uses a 256-sample window at Fs=8 kHz. What is the window duration in milliseconds?

5. Frequency localization

Which generally improves frequency localization: a longer or shorter STFT window? What does it cost?

6. Power spectrogram

What quantity is visualized by a power spectrogram?