ECEN 4/5632        Theory and Applications of Digital Filters         Fall 2026

9-24-26                                                                                                                                                            P. Mathys

Name:

Midterm 1

“On my honor, as a University of Colorado at Boulder student, I have neither given nor received unauthorized assistance on this work” (see full Honor Code at http://www.colorado.edu/academics/honorcode)

- Closed book, notes, electronics basic scientific calculator ok.

- Covers Lectures 1 to 8 and Problem Sets 1 to 4.

Instructions. Show enough work to support every numerical answer. State ROCs explicitly. Label units and discrete-time frequencies. Unsupported answers may receive little credit.

| Problem | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Points | /20 | /18 | /24 | /20 | /18 |

Total:                  /100

| Convolution and Transforms | yn=kxkh[n-k] Xz=nxnz-n Xejω=nxne-jωn |
| --- | --- |
| Frequency Mapping |  |
| Quantization |  |
| Filter Design |  |

Left blank

Problem 1 Convolution and LTI Frequency Response, 20 points, approx. 14 minutes

Let xn=δn+2δn-1-δ[n-2] and hn=δn-δ[n-1].

(a) [2 points] Predict the index range on which yn=x[n]*h[n] can be non-zero.

(b) [4 points] Compute every non-zero sample of y[n].

(c) [10 points] Find Hejω. For the input signal voltage vn=3cos⁡(πn2+π6), find the steady-state output in cosine form.

(d) [4 points] PyTorch conv1d computes cross-correlation. For an input window ordered [xn-1,xn], give the two fixed kernel weights that produce yn=xn-x[n-1]. Explain the reversal.

Left blank

Problem 2 Sampling Aliasing and Quantization, 18 points, approx. 12 minutes

A signal xat=2cos2π∙1500t+cos⁡(2π∙6500t) is sampled at fs=8000 Hz.

(a) [6 points] Write x[n] using discrete-time angular frequencies reduced to the interval -π,π.

(b) [5 points] Show that the two analog components produce the same discrete-time cosine. Simplify x[n]. What information has been lost?

(c) [3 points] If the 1500 Hz component is desired and the 6500 Hz component is unwanted, state a defensible anti-alias filter requirement needed before sampling.

(d) [4 points] A 10-bit uniform quantizer spans -1 V to +1 V. Find ∆ and estimate the full-scale sine SQNR in dB.

Left blank

Problem 3 Inverse z-Transform and ROC, 24 points, approx. 18 minutes

Problem 4 Difference Equation Poles Zeros and Stability, 20 points, approx. 14 minutes

Problem 5 FIR Window Design and conv1d, 18 points, approx. 12 minutes

Problem 1. Consider the signal

yt=15x(-2t-3)

shown in the figure on the right.

(a) Does y(t) have an odd portion, yo(t)? If so, determine and carefully sketch and label yo(t). Otherwise, explain why no odd portion exists.

(b) Determine and carefully sketch and label the original signal x(t).

Left blank

Problem 2. A discrete time system is described by

yn=12k=-∞∞xkδn-k+δn+k

(a) Explain what this system does.

(b) Is this system BIBO stable? Justify your answer.

(c) Is the system linear? Justify your answer.

(d) Is the system memoryless? Justify your answer.

(e) Is the system causal? Justify your answer.

(f) Is the system time-invariant? Justify your answer.

Left blank

Problem 3. The figure on the right shows a cascade of three causal LTI systems. The impulse response h1n is

h1n=un-2un-1+u[n-2]

and the overall impulse response h[n] is shown in the stem plot on the right.

(a) Find the impulse response h2[n].

h2[n]=

(b) Find, sketch and label the response of the overall system to the input

xn=δn+δ[n-1]

Left blank

Problem 4. The input x1(t) and corresponding output y1(t) of a causal CT LTI system are shown below.

(a) Determine the unit step response s(t) and the unit impulse response h(t) of the CT LTI system. Show all your computations and/or reasonings.

(b) If an input x2(t), as shown on the right, is applied to the above CT LTI system, what is the corresponding output y2(t)? Carefully plot and label y2(t).

Left blank