# Week 5 Homework Quiz Solutions

Week 5: Filter Properties

Lecture 9: Time/Frequency Responses and Group Delay

Lecture 10: Linear Phase, Generalized Phase, and ML Intersections

# Lectures 9 and 10 Homework and Weekly Quiz

All mathematical expressions use raw $...$ and $$...$$ delimiters.

## Homework for Lecture 9

### Problem 1; Pole-zero geometry

For

H(z)=1−0.8z−11−0.5z−1,

(a) identify the zero and pole; (b) explain geometrically why |H(ejω)| is the product of zero-vector lengths divided by the product of pole-vector lengths; (c) predict qualitatively whether the magnitude is larger near ω=0 or ω=π.

### Problem 2: Phase delay and group delay

Let

ϕ(ω)=−3ω+0.4sinω.

Compute the phase delay

τp(ω)=−ϕ(ω)ω

for ω≠0 and the group delay

τg(ω)=−dϕ(ω)dω.

Explain why the system is not linear phase.

### Problem 3: First-order all-pass

For real |a|<1,

Hap(z)=z−1−a1−az−1.

Show that

|Hap(ejω)|=1

for all ω. Locate the pole and zero and explain the reciprocal relationship.

### Problem 4: ML feature distortion

An ECG classifier uses QRS peak timing and onset slope as features. Explain how a causal IIR with nonlinear phase can alter those features even if its magnitude response removes only an irrelevant high-frequency noise band. Propose one offline and one streaming-safe mitigation.

## Homework for Lecture 10

### Problem 5: Symmetry and linear phase

A real FIR filter has length N=M+1 and satisfies

h[n]=h[M−n].

Show that its frequency response can be written as

H(ejω)=e−jωM/2A(ω),

where A(ω) is real. State the group delay.

### Problem 6: FIR Types I and IV

For each case below, identify the FIR type and state any forced zeros at ω=0 or ω=π:

- N=9, symmetric.

- N=8, symmetric.

- N=9, antisymmetric.

- N=8, antisymmetric.

Then explain why a Type II filter cannot realize a high-pass response with nonzero gain at ω=π.

### Problem 7: Four-zero constellation

A real linear-phase FIR has a zero at

z0=0.7ej0.6.

List the other zeros implied by real coefficients and linear-phase reciprocal symmetry.

### Problem 8: Symmetry-constrained Conv1d

Describe a PyTorch parameterization that guarantees a trainable odd-length Conv1d kernel remains symmetric during training. Explain what inductive bias this imposes and what flexibility it removes.

# Weekly Quiz -- Student Version

### Q1

For

H(z)=Ki​(z−zi)k​(z−pk),

what geometric quantities determine |H(ejω)| when z=ejω?

### Q2

State the definitions of phase delay and group delay.

### Q3

What property distinguishes an all-pass filter from an ordinary magnitude-shaping filter?

### Q4

Why can two filters with nearly identical magnitude responses produce different transient waveforms?

### Q5

A real FIR has N=21 symmetric taps. What is its group delay?

### Q6

Which FIR type is even-length and symmetric, and what forced endpoint zero does it have?

### Q7

A complex zero z0 of a real linear-phase FIR is neither real nor on the unit circle. What other three zeros accompany it?

### Q8

Does a randomly initialized nn.Conv1d kernel automatically have linear phase? Briefly justify.

# Solutions

## Homework Solutions

### Solution 1

The zero is at z=0.8 and the pole is at z=0.5. Evaluating on the unit circle gives

H(ejω)=Ki​(ejω−zi)k​(ejω−pk).

Therefore

|H(ejω)|=|K|i​|ejω−zi|k​|ejω−pk|.

Each magnitude is a Euclidean vector length in the z-plane. Near ω=0, the unit-circle point is z=1, which is closer to the zero at 0.8 than to the pole at 0.5, so the numerator is relatively small. Near ω=π, the point is z=−1 and the distance ratio is less suppressed. Direct checks give

|H(1)|=0.20.5=0.4,

|H(−1)|=1.81.5=1.2.

Thus the magnitude is larger near ω=π.

### Solution 2

τp(ω)=3−0.4sinωω.

Also,

dϕdω=−3+0.4cosω,

so

τg(ω)=3−0.4cosω.

Because τg(ω) varies with ω, the phase is not linear in frequency.

### Solution 3

On the unit circle,

Hap(ejω)=e−jω−a1−ae−jω.

The squared numerator magnitude is

|e−jω−a|2=1+a2−2acosω,

and the squared denominator magnitude is

|1−ae−jω|2=1+a2−2acosω.

Hence

|Hap(ejω)|=1.

The pole is at z=a and the zero is at z=1/a. For |a|<1, the pole is inside the unit circle and the reciprocal zero is outside.

### Solution 4

Nonlinear phase means frequency-dependent group delay, so different spectral components of the QRS transient can move by different amounts. That can change peak time, onset slope, and morphology even when the magnitude response removes only nominal noise. Offline mitigation: use a validated forward-backward zero-phase method while accounting for the squared magnitude response. Streaming-safe mitigation: use a linear-phase FIR or train/evaluate with the exact causal preprocessing that will be available at deployment.

### Solution 5

Pair symmetric samples about M/2. The frequency response can be factored as

H(ejω)=e−jωM/2A(ω),

where A(ω) is real. Therefore the phase is linear apart from possible π jumps caused by sign changes in A(ω), and the group delay is

τg=M2=N−12.

### Solution 6

- N=9, symmetric: Type I; no endpoint zero is forced by symmetry alone.

- N=8, symmetric: Type II; forced zero at ω=π, equivalently z=−1.

- N=9, antisymmetric: Type III; forced zeros at ω=0 and ω=π, equivalently z=1 and z=−1.

- N=8, antisymmetric: Type IV; forced zero at ω=0, equivalently z=1.

A Type II response must satisfy H(ejπ)=0, so it cannot have nonzero high-pass gain at Nyquist.

### Solution 7

Given

z0=0.7ej0.6,

real coefficients imply the conjugate

z0*=0.7e−j0.6.

Linear-phase reciprocal symmetry also implies

1z0=10.7e−j0.6

and

1z0*=10.7ej0.6.

Thus the four-zero constellation is

z0, z0*, 1z0, 1z0*.

### Solution 8

For odd length N=2R+1, learn only R+1 independent parameters and construct

h=[a0,a1,…,aR−1,aR,aR−1,…,a1,a0].

In PyTorch, concatenate the learned half, center tap, and a reversed copy. The resulting kernel is symmetric for every optimizer step, so it has exact generalized linear phase. This imposes a phase-preserving inductive bias and roughly halves the number of independent coefficients, but it prevents the model from learning arbitrary nonlinear-phase kernels.

## Weekly Quiz Solutions

### A1

|H(ejω)|=|K|i​|ejω−zi|k​|ejω−pk|.

The relevant geometric quantities are the vector lengths from each zero and pole to the unit-circle point ejω.

### A2

τp(ω)=−∠H(ejω)ω,

τg(ω)=−ddω∠H(ejω).

### A3

An all-pass filter satisfies

|H(ejω)|=1

for all ω while changing phase and group delay.

### A4

Their phase responses can differ. Frequency-dependent group delay changes the relative arrival times of frequency components and can reshape a transient even when magnitudes are nearly the same.

### A5

τg=21−12=10 samples.

### A6

Type II. It is even-length and symmetric and has a forced zero at

ω=π,

or z=−1.

### A7

z0*, 1z0, 1z0*.

### A8

No. A generic nn.Conv1d learns unconstrained kernel coefficients, so symmetry or antisymmetry is not guaranteed. Exact linear phase must be imposed by parameterization, constraints, or a suitable regularizer/architecture.