ECEN 4/5632         Theory and Application of Digital Filters          Fall 2026

10-2-26                                                                                                                                             P. Mathys

Problem Set 6: Solution

1. Conv1d and the sliding dot product

Given the DSP convolution sum yDSPn=khkx[n-k] and PyTorch’s cross-correlation convention ytorchn=kwkx[n+k]:

(a) Derive the relationship wk=h[M-k] (where h has M+1 taps, indices 0,…,M) that makes F.conv1d reproduce the textbook causal FIR convolution, and state the required left-padding amount.

(b) Implement both, the NumPy convolution np.convolve and the equivalent F.conv1d computation for a Hamming-windowed FIR lowpass filter with ωc=π3 and length N=51 on a chirp signal (chirp_001.wav). Report the maximum absolute difference between the two outputs (you should get something on the order of 10-6 to 10-7 in float32). Show evidence that your filter works as desired (e.g., by recording the filtered chirp signal as .wav-file and using a program like Audacity to generate a spectrogram). Show all your Python and PyTorch code in your solution.

Python code, part 1

Lowpass filter, time and frequency domain response

Python code, part 2

Note that the maximum error between the numpy.convolve() and the torch.nn.functional.conv1d() is only about 3.59×10-7.

Time waveforms and spectrograms from Audacity, original Chirp signal:

After DSP low-pass filtering:

After conv1d low-pass filtering:

(c) A colleague says “PyTorch’s conv1d does convolution backwards, so it’s a bug.” Give a one-paragraph rebuttal, using the ML viewpoint discussed in lecture.

Rebuttal: It's not a bug, just a different (and for machine learning purposes, more natural) convention. Mathematically, convolution and cross-correlation differ only by flipping the kernel, w[k] = h[M-k], exactly as derived in part (a), so no information or capability is lost, and the two are trivially interconvertible. In classical DSP the kernel h[k] is a fixed, physically meaningful impulse response, derived from a difference equation or an analog prototype, so the flip in the definition matters and must be tracked carefully. In deep learning, however, the kernel w[k] is a set of learned parameters with no prior physical meaning: gradient descent will happily learn whichever set of numbers minimizes the loss, whether the operation is called “convolution” or “correlation.” Since training and inference both consistently use the same cross-correlation definition, and autograd computes the correct corresponding gradients automatically, there is no mismatch anywhere in the pipeline. The “backwards” framing only becomes an issue when reusing a fixed, externally designed filter, like the FIR filter in part (b), inside a conv1d call, and even then, as shown above, it is a one-line fix: flip the kernel and left/right-pad by M, not a bug.

2. Pole-zero geometry

For

Hz=1-0.8z-11+0.5z-1

(a) Identify the zero and the pole.

(b) Explain geometrically why H(ejω| is the product of zero-vector lengths divided by the product of pole-vector lengths.

(c) Predict qualitatively whether the magnitude is larger near ω=0 or ω=π.

3. Phase delay and group delay

Let Hejω=Hejωejϕ(ω) with ϕω=-3ω+0.4sin⁡(ω).

(a) Compute the phase delay

τpω=-ϕ(ω)ω

for ω≠0.

(b) Compute the group delay

τgω=-dϕωdω.

(c) Explain why the system is not linear phase.

4. First order allpass

For real a<1,

Hapz=z-1-a1-az-1.

Show that

|Hapejω|=1

for all ω. Locate the pole and zero and explain the reciprocal relationship.

5. Inverse system properties (ECEN 5632 Problem)

If the system function H(z) of an LTI system has a pole-zero plot as shown on the right and the system is causal, can the inverse system Hi(z), where HzHiz=1, be both causal and stable? Clearly justify your answer.