ECEN 4/5632         Theory and Application of Digital Filters          Fall 2026

9-28-26                                                                                                                                             P. Mathys

Quiz 4

1. FIR filter

A real-valued FIR filter has N=31 symmetric taps. What is its constant group delay?

2. Gibbs phenomenon

In one sentence explain the Gibbs phenomenon in windowed FIR design.

3. Butterworth filter

For a Butterworth lowpass, what is the magnitude at Ω=Ωc in dB?

4. DT IIR filter

A desired DT cutoff frequency is ωc=0.6π. Should the analog prototype for the DT filter be designed directly at Ωc=0.6πT? Explain briefly.

5. Pole-zero geometry

For Hz=Ki(z-zi)k(z-zk), what geometric quantities determine |Hejω| when z=ejω?

6. Filter comparison

Why can two filters with nearly identical  magnitude responses produce different transient waveforms?