ECEN 4/5632         Theory and Application of Digital Filters          Fall 2026

9-28-26                                                                                                                                             P. Mathys

Quiz 4

1. FIR filter [1 point]

A real-valued FIR filter has N=31 symmetric taps. What is its constant group delay?

τg=(N-1)2=15 samples.

2. Gibbs phenomenon [2 points]

In one sentence explain the Gibbs phenomenon in windowed FIR design.

Abrupt truncation of an ideal response creates oscillatory ripple near the frequency discontinuity; increasing length narrows the ripple region, but does not drive the peak overshoot to zero.

3. Butterworth filter [1 point]

For a Butterworth lowpass, what is the magnitude at Ω=Ωc in dB?

|HcjΩc|=12    =>    -3.01 dB

4. DT IIR filter [2 points]

A desired DT cutoff frequency is ωc=0.6π. Should the analog prototype for the DT filter be designed directly at Ωc=0.6πT? Explain briefly.

No, the bilinear transform warps frequency. Prewarp first:  Ωc=2Ttan⁡(0.6π2).

5. Pole-zero geometry [2 points]

For Hz=Ki(z-zi)k(z-zk), what geometric quantities determine |Hejω| when z=ejω?

Hejω=|K|i|ejω-zi|k|ejω-pk|. The relevant geometric quantities are the vector lengths from each zero and pole to the point ejω on the unit circle.

6. Filter comparison [2 points]

Why can two filters with nearly identical  magnitude responses produce different transient waveforms?

Their phase responses can differ. Frequency-dependent group delay changes the relative arrival times of frequency components and can reshape a transient, even when magnitudes are nearly the same.