ECEN 4/5632         Theory and Application of Digital Filters          Fall 2026

10-9-26                                                                                                                                             P. Mathys

Problem Set 7: Solution

1. FIR filter classification & frequency response derivation

Given four causal discrete-time FIR systems with impulse responses defined for 0≤n≤M:

1. h1[n] = [1,3,5,3,1]

2. h2[n] = [2,-1,4,4,-1,2]

3. h3[n] = [1,2,0,-2,-1]

4. h4[n] = [-1,3,-3,1]

(a) Classify each system into its corresponding FIR Type (I, II, III, or IV). Specify the filter order M and sequence length N for each.

(b) For h2[n], derive the compact closed-form expression for its frequency response H2(ejω). Express H2(ejω) in the form Aωe-jM2 where A(ω) is a real-valued trigonometric sum.

(c) For h3[n], derive H3(ejω) and show that it exhibits a constant 90o (π/2 rad) phase shift relative to the linear phase delay term e-jM/2 .

2. Zero Constellation Symmetry & System Synthesis

(a) Prove that if z0=rejθ (with r≠1 and θ≠0,π) is a zero of a causal, real-coefficient linear-phase FIR filter H(z), then z0, z0*, 1z0, and 1z0* must also be zeros of H(z) with equal multiplicity.

(b) A Type I FIR filter of order M=4 (N=5) with real impulse response h[n] has a complex zero at:

z1=12ejπ/3

i. List all four zeros of H(z) in both polar and Cartesian forms.

ii. Derive the normalized system transfer function H(z) given that H(ej0)=1.

iii. Determine the explicit impulse response sequence h[n] for n=0,1,2,3,4.

3. System interconnections and composite types

Consider two causal linear-phase FIR filters: HA(z) of Type II with order MA, and HB(z) of Type IV with order MB.

(a) Determine the FIR Type (I, II, III, or IV) of the cascaded system HC(z)=HAz∙HB(z). Express the overall order MC in terms of MA and MB, and justify the symmetry of hC[n].

(b) Assume MA=MB=M. Determine whether the parallel interconnection HD(z)=HA(z)+HB(z) maintains strict generalized linear phase. Support your answer mathematically by evaluating the phase of HD(ejω).

(c) Can a parallel combination of a Type I filter H1(z) and a Type III filter H3(z) with identical delay τ=M/2 yield a linear phase system? Explain.

4. Filter flowgraph (ECEN 5632 Problem)

Consider the filter flowgraph shown in the figure below:

The z-1 cells are delay cells and the bubbles with numbers in them are multipliers with the numbers giving the multiplication factors. The bubbles with + signs are adders, the branch labeled x[n] is the input of the filter and the branch labeled yn is the output of the filter.

(a) Determine the unit impulse response h[n] of the filter shown above and display the result in the form of a labeled stem plot. What type of a filter is it?

(b) What would be the main advantage of implementing a filter in this way? Are there any disadvantages?

(c) Could you implement a Type III and/or Type IV FIR filter in this way? If yes, what would you have to change, if no, why not?