ECEN 4/5632         Theory and Application of Digital Filters          Fall 2026

9-18-26                                                                                                                                             P. Mathys

Problem Set 4: Solution

1. Difference equation to H(z)

A system obeys y[n]-1.4y[n-1]+0.45y[n-2]=x[n]+x[n-1].

a) Find H(z).

b) Determine all zeros and poles of H(z).

c) Determine the ROC if the system is causal.

d) Is the causal system BIBO stable?

2. Geometric frequency-response reasoning

Consider H(z)=(1+z⁻¹)/(1-0.8z⁻¹).

a) Determine the poles and zeros of H(z).

b) Use a geometric method to compare |Hejω| near ω=0 and ω=π. Show a labeled sketch of the geometric evaluation.

c) What is your conclusion about the qualitative filter type of this H(z)?

3. 600 Hz notch filter design

At fs=9000 Hz, design the pole-zero locations of a second-order notch at 600 Hz using zeros on the unit circle and poles at radius r=0.96 with the same angles ω0.

a) Give ω₀, the two zero locations and the two pole locations.

b) Explain what increasing r to 0.99 would do.

4. Ideal lowpass FIR filter and causal realization

An ideal discrete-time lowpass filter has

Hd(ejω)=1,|ω|≤ωc,0,ωc<|ω|≤π.

a) Use the inverse DTFT to determine the unit impulse response hd[n]

b) Sketch and label hd[n] for ωc=π/3 and n in the range [-8,8]. What is the value of hd[0]?

c) Explain why this ideal impulse response cannot be implemented exactly as a real-time FIR filter.

d) For an odd-length filter with N=61 taps, state the shift required to make a symmetric truncated version causal and give its constant group delay.

5. Window tradeoff

A student designs two 51-tap lowpass filters with the same cutoff. Filter A uses a rectangular window; Filter B uses a Hamming window.

a) Which filter should have the narrower transition band?

b) Which filter should have lower stopband sidelobes?

c) Give approximate first-sidelobe levels for the two windows.

d) Explain why merely increasing the rectangular-window length does not remove the Gibbs overshoot near a discontinuity.

6. Symmetry and linear phase

Let a real-valued FIR filter have length N and satisfy

h[n]=h[N-1-n].

a) Show that its frequency response can be written in the form

H(ejω)=e-jω(N-1)/2A(ω),

where A(ω) is real-valued.

b) What is the group delay where A(ω)≠0? Why can this be useful in machine learning in a feature-extraction pipeline?

7. Pole radius and time-domain decay (ECEN 5632 Problem)

A causal system has a complex pole pair p=r e±jθ. Explain why the corresponding impulse-response terms decay roughly as rⁿ and why moving r toward 1 simultaneously increases ringing duration and sharpens the frequency-domain resonance. Connect your argument to geometric distance from the unit circle.