ECEN 4/5632         Theory and Application of Digital Filters          Fall 2026

9-25-26                                                                                                                                             P. Mathys

Problem Set 5: Solution

1. Butterworth magnitude response

For an N=4 analog Butterworth lowpass filter,

|HcjΩ​2=11+ΩΩc2N.

Compute |HcjΩ| in dB at (i) Ω=Ωc and (ii) Ω=2Ωc.

2. Butterworth order and cutoff behavior

The Butterworth magnitude-squared response is

|HcjΩ​2=11+ΩΩc2N.

(a) Derive the standard Butterworth low-pass order formula that guarantees a passband attenuation ≤Ap in dB at Ωp in rad/sec and a stopband attenuation ≥As in dB at Ωs in rad/sec:

N≥log1010As10-110Ap10-12log10(ΩsΩp)

Hints: Write Hc(jΩp)2 and HcjΩs2 in terms of N, form a ratio, and solve for N.

(b) For Ap=1 dB at Ωp=1 rad/sec and As=40 dB at Ωs=3 rad/sec, compute the minimum required order N. Round up to the nearest integer and explain why you round up rather than down.

(c) If you change Ωs in part (b) to Ωs=2 rad/sec, a 33% change in Ωs, how does N change?

3. Stability under the bilinear transform

The bilinear transform is

z=1+sT21-sT2.

(a) For T=0.1 sec and analog pole

sp=-2+j3,

compute the mapped digital pole and verify that it lies inside the unit circle.

(b) Determine the warped digital frequency ω form the analog frequency Ω using

ω=2tan-1(ΩT2)

and verify your result in (a).

4. Prewarping design exercise

You need a digital Butterworth low-pass filter with T=1 (i.e., fs=1 Hz, normalized) and a digital passband edge at ωp=0.3π where the response should be at -1 dB, and a digital stopband edge at ωs=0.6π where the response should be at least -30 dB down.

(a) Prewarp both edges to analog frequencies Ωp and Ωs using

Ω=2Ttanω2.

(b) Using your formula from problem 2, compute the required Butterworth order N.

(c) In Python, design the analog Butterworth prototype at the prewarped Ωp (adjusted to Ωc, comment on why and how you did it) using scipy.signal.butter(N,Wc,analog=True), then apply scipy.signal.bilinear (or bilinear_zpk) to obtain the digital filter. Plot and label the digital magnitude response and confirm the -1 dB and -30 dB points land at (approximately) 0.3π and 0.6π.

(d) Compute, plot and label the group delay τg(ω) over 0≤ω≤π using scipy.signal.group_delay.

(e) Repeat the design in (c) without prewarping (i.e., using Ωp=ωp, Ωs=ωs directly as analog frequencies) and show on the same plot how far the resulting digital cutoff misses the intended target. Quantify the error in normalized frequency.

5. Window method as frequency-domain convolution (ECEN 5632 Problem)

The window method result Hwejω=12πHd(ejω)*W(ejω) is a periodic convolution.

(a) Explain intuitively why a narrower main lobe of W(ejω) leads to a sharper transition band in Hw(ejω).

(b) Explain intuitively why lower sidelobes in W(ejω) lead to lower ripple in Hw(ejω), and why these two goals (narrow main lobe, low sidelobes) are in tension for a fixed window length N. (Hint: think about the time-domain taper's smoothness versus its duration.)