# Week 1 Homework and Weekly Quiz

Signals • LTI Systems • Convolution • Eigenfunctions • DTFT

Instructor version with complete solutions

| Audience | ECE seniors + first-year graduate students |
| --- | --- |
| Suggested homework effort | 90–120 minutes core + 20–30 minutes graduate stretch |
| Suggested quiz time | 15 minutes |
| Text alignment | Oppenheim & Schafer, Discrete-Time Signal Processing — LTI systems and DTFT topics |

Teaching intent. The core problems emphasize mathematical reasoning and indexing discipline. The graduate-stretch items ask for proof, convergence, or generalization beyond a single numerical example.

# Part I — Homework Problems

Show enough work that another engineer can follow your indexing, assumptions, and reasoning. Use Python only where a problem explicitly asks for it or as a check after the analytic work.

1. Impulse and step representations  [8 points]

The sequence x[n] is x[−1]=3, x[0]=−2, x[2]=4, and zero elsewhere.

a) Express x[n] as a sum of scaled, shifted unit samples.

b) Let v[n]=u[n+1]−u[n−2]. List the nonzero samples of v[n] and sketch it.

c) Express δ[n+2] using two shifted unit steps.

2. System-property classification  [16 points]

For each system below, determine whether it is linear, time invariant, causal, and BIBO stable. Give a one- or two-sentence justification for every “no.”

A. y[n]=2x[n−2]

B. y[n]=x²[n]

C. y[n]=n x[n]

D. y[n]=x[n]+0.5x[n−1]

3. Finite convolution and support  [10 points]

Let x[n]={1,−1,2} for n=0,1,2 and h[n]={2,1} for n=0,1.

a) Predict the index range over which y[n]=x[n]*h[n] can be nonzero.

b) Compute the full convolution by hand.

c) Verify the result with np.convolve.

4. Impulse response, causality, stability, and step response  [12 points]

An LTI system has h[n]=(0.75)^n u[n].

a) Is the system causal? Explain from h[n].

b) Is the system BIBO stable? Show the relevant sum.

c) Find the response to x[n]=u[n]. Give a closed-form expression for n≥0.

d) What is the limiting output value as n→∞?

5. Sinusoid through an LTI system  [12 points]

An LTI system has H(e^{jω})=1+0.5e^{−jω}. The input is x[n]=2cos(πn/3+π/4).

a) Evaluate H(e^{jπ/3}) in rectangular form.

b) Find its magnitude and phase.

c) Write the steady-state output y[n] in cosine form.

6. DTFT shifting and a spectral zero  [10 points]

Let x[n]=(0.5)^n u[n] and y[n]=x[n−2]−x[n].

a) Find X(e^{jω}).

b) Use the time-shift property to find Y(e^{jω}).

c) Evaluate Y(e^{j0}) and explain what that value says about the sum of y[n].

7. Difference equation to frequency response  [12 points]

A causal system satisfies y[n]−0.8y[n−1]=x[n]−x[n−1], with zero initial conditions.

a) Derive H(e^{jω}).

b) Evaluate H at ω=0 and ω=π.

c) Based only on those values and the structure of the numerator, is the system more “low-pass-like” or “high-pass-like”? Explain.

d) Use scipy.signal.freqz to check your reasoning.

8. Direct DTFT evaluation  [10 points]

Let x[n]={1,2,−1} for n=0,1,2.

a) Write X(e^{jω}) explicitly.

b) Evaluate X(e^{jω}) at ω=0, π/2, and π.

c) Use the 2π-periodicity property to state X(e^{j5π/2}) without recomputing the sum.

9. BIBO stability proof for an LTI system  [10 points]  — Graduate stretch

Assume |x[n]|≤B_x for all n and that Σ_k |h[k]| < ∞. Starting from y[n]=Σ_k x[k]h[n−k], prove that y[n] is bounded for all n. State an explicit bound B_y.

Then explain in one sentence why h[n]=(0.99)^n u[n] is stable even though its impulse response lasts forever.

# Part II — Homework Solutions

1. Impulse and step representations

a) x[n]=3δ[n+1]−2δ[n]+4δ[n−2]. The sign inside δ[n−n0] is tied to the location n0.

b) u[n+1] turns on at n=−1 and u[n−2] turns on at n=2. Therefore v[n]=1 for n=−1,0,1 and 0 otherwise.

c) δ[n+2]=u[n+2]−u[n+1].

2. System-property classification

A. y[n]=2x[n−2]: linear, time invariant, causal, BIBO stable. It is a fixed gain and fixed delay.

B. y[n]=x²[n]: nonlinear, time invariant, causal, BIBO stable. Squaring violates superposition, but |x|≤B implies |y|≤B².

C. y[n]=n x[n]: linear, time varying, causal, not BIBO stable. A bounded input x[n]=1 produces y[n]=n, which is unbounded.

D. y[n]=x[n]+0.5x[n−1]: linear, time invariant, causal, BIBO stable. Its impulse response is δ[n]+0.5δ[n−1], whose absolute sum is 1.5.

3. Finite convolution and support

a) x is supported on 0…2 and h on 0…1, so y is supported on 0…3. The full-convolution length is 3+2−1=4.

b) y[0]=1·2=2. y[1]=1·1+(−1)·2=−1. y[2]=(−1)·1+2·2=3. y[3]=2·1=2. Therefore y={2,−1,3,2}.

c) np.convolve([1,-1,2],[2,1]) returns [2, -1, 3, 2].

4. Impulse response, causality, stability, and step response

a) Causal: h[n]=0 for n<0 because of u[n].

b) Stable: Σ_n |h[n]|=Σ_{n=0}^∞(0.75)^n=1/(1−0.75)=4<∞.

c) For n≥0, y[n]=Σ_{k=0}^n (0.75)^k = (1−0.75^{n+1})/(1−0.75)=4(1−0.75^{n+1}). For n<0, y[n]=0.

d) lim_{n→∞} y[n]=4.

5. Sinusoid through an LTI system

At ω0=π/3, e^{−jπ/3}=1/2−j√3/2. Therefore H=1+0.5(1/2−j√3/2)=1.25−j0.4330.

|H|=√(1.25²+0.4330²)=√1.75≈1.3229. ∠H≈−0.3335 rad (−19.1°).

For a real sinusoid through a real LTI system, y[n]=2|H|cos(πn/3+π/4+∠H). Thus y[n]≈2.6458 cos(πn/3+0.4519).

6. DTFT shifting and a spectral zero

a) X(e^{jω})=1/(1−0.5e^{−jω}) because |0.5|<1.

b) x[n−2] ↔ e^{−j2ω}X(e^{jω}), so Y(e^{jω})=(e^{−j2ω}−1)/(1−0.5e^{−jω}).

c) Y(e^{j0})=(1−1)/(1−0.5)=0. Since Y(e^{j0})=Σ_n y[n] when the sum converges, the samples of y[n] sum to zero.

7. Difference equation to frequency response

Taking the DTFT (or substituting a complex exponential): Y−0.8e^{−jω}Y=X−e^{−jω}X.

Therefore H(e^{jω})=(1−e^{−jω})/(1−0.8e^{−jω}).

At ω=0, H=0. At ω=π, H=(1−(−1))/(1−0.8(−1))=2/1.8≈1.111.

The zero at DC and nonzero response at π make the system high-pass-like. A freqz plot should confirm stronger high-frequency response.

8. Direct DTFT evaluation

a) X(e^{jω})=1+2e^{−jω}−e^{−j2ω}.

b) X(1)=X(e^{j0})=1+2−1=2. At ω=π/2: e^{−jπ/2}=−j and e^{−jπ}=−1, so X=1−2j+1=2−2j. At ω=π: X=1−2−1=−2.

c) 5π/2=π/2+2π, so X(e^{j5π/2})=X(e^{jπ/2})=2−2j.

9. BIBO stability proof for an LTI system

|y[n]|=|Σ_k x[k]h[n−k]|≤Σ_k |x[k]| |h[n−k]|≤B_x Σ_k |h[n−k]|.

A shift does not change the absolute sum, so Σ_k |h[n−k]|=Σ_m |h[m]|. Therefore |y[n]|≤B_x Σ_m |h[m]|≡B_y<∞ for every n.

For h[n]=(0.99)^n u[n], the absolute sum is 1/(1−0.99)=100, which is finite. Infinite duration does not imply instability; lack of absolute summability does.

# Part III — Weekly Quiz (Student Version)

Suggested time: 15 minutes. Total: 20 points. Calculator not required.

1. [2 pts] Which sequence equals δ[n−3]?

A. u[n−3]−u[n−4]    B. u[n+3]−u[n+2]    C. u[n−2]−u[n−3]    D. u[n]−u[n−3]

Answer: ________________________________________________________________________________________

_________________________________________________________________________________________________________

2. [2 pts] The system y[n]=x[n−2]+x[n+2] is:

A. linear, time invariant, causal    B. linear, time invariant, noncausal    C. nonlinear, time invariant    D. linear, time varying

Answer: ________________________________________________________________________________________

_________________________________________________________________________________________________________

3. [2 pts] A length-5 sequence convolved with a length-4 sequence has what full-convolution length?

Show the number and one sentence of reasoning.

Answer: ________________________________________________________________________________________

_________________________________________________________________________________________________________

4. [3 pts] Let x[n]={1,2} and h[n]={1,−1}, both beginning at n=0. Compute y[n]=x*h.

Answer: ________________________________________________________________________________________

_________________________________________________________________________________________________________

5. [2 pts] Why are complex exponentials called eigenfunctions of LTI systems?

Answer in one or two sentences.

Answer: ________________________________________________________________________________________

_________________________________________________________________________________________________________

6. [2 pts] If X(e^{jω}) is a DTFT, what is X(e^{j(ω+2π)})?

Explain why in one line.

Answer: ________________________________________________________________________________________

_________________________________________________________________________________________________________

7. [4 pts] For y[n]=0.5y[n−1]+x[n], derive H(e^{jω}). Then evaluate H at ω=0 and ω=π.

Answer: ________________________________________________________________________________________

_________________________________________________________________________________________________________

8. [3 pts] Based on Question 7, does the system emphasize low or high normalized frequencies? Explain without plotting.

Answer: ________________________________________________________________________________________

_________________________________________________________________________________________________________

# Part IV — Weekly Quiz Solutions

1. Unit-sample identity

A. δ[n−3]=u[n−3]−u[n−4].

2. System properties

B. The rule is linear and a fixed combination of shifts, so it is time invariant. It is noncausal because y[n] depends on the future value x[n+2].

3. Convolution length

5+4−1=8 samples. Full linear convolution adds the support lengths minus one.

4. Short convolution

[1,2]*[1,−1]=[1,1,−2]. Specifically y[0]=1, y[1]=−1+2=1, y[2]=−2.

5. Eigenfunction meaning

For input e^{jωn}, every LTI system produces H(e^{jω})e^{jωn}. The function’s shape/frequency is unchanged; only a complex scale factor is applied.

6. DTFT periodicity

X(e^{j(ω+2π)})=X(e^{jω}) because e^{−j(ω+2π)n}=e^{−jωn}e^{−j2πn}=e^{−jωn} for integer n.

7. First-order frequency response

Rearrange: y[n]−0.5y[n−1]=x[n]. Thus H(e^{jω})=1/(1−0.5e^{−jω}). At ω=0, H=2. At ω=π, H=1/(1+0.5)=2/3.

8. Qualitative interpretation

Low frequencies are emphasized because the response magnitude is larger at ω=0 than at ω=π. The denominator becomes smallest when e^{−jω} is near +1.

## Instructor scoring notes

For Questions 5, 6, and 8, award most credit for correct reasoning even if notation is imperfect. On Question 7, separate algebra/setup credit from the two endpoint evaluations. The quiz intentionally checks conceptual understanding more than arithmetic speed.