# ECEN 4/5632 Problem Set 3 Solutions

## from ClaudeAI

## 1. Ideal Sinc Reconstruction

Given: xn=δn+2δn−1, sample period T.

### a) Reconstructed signal

Ideal bandlimited reconstruction replaces each sample with a shifted, scaled sinc:

xrt=n​xn sinc​t−nTT=sinc​tT+2 sinc​t−TT

### b) Verify sample values

xr0=sinc0+2 sinc−1=1+20=1✓

(using sinc−1=sin−π/−π=0)

xrT=sinc1+2 sinc0=0+21=2✓

This confirms the key sinc property: sinck=0 for nonzero integers k, and sinc0=1, so the reconstruction passes exactly through each sample.

### c) Value at t=T/2

xrT/2=sinc0.5+2 sinc−0.5

sinc0.5=sinπ/2π/2=2π,  sinc−0.5=sin−π/2−π/2=2π

xrT/2=2π+22π=6π≈1.9099

## 2. Zero-Order Hold Frequency Response

Given: hZOHt=ut−ut−T, a rectangular pulse of height 1 on 0,T.

### a) Direct integration

HZOHjΩ=0Te−jΩt dt=e−jΩt−jΩ0T=1−e−jΩTjΩ

### b) Separate into linear-phase and magnitude terms

Factor out the "half-delay" phase:

HZOHjΩ=e−jΩT/2⋅ejΩT/2−e−jΩT/2jΩ=e−jΩT/2⋅2sinΩT/2Ω

Writing the magnitude term as a normalized sinc:

HZOHjΩ=T sinc​ΩT2πe−jΩT/2

- Linear-phase term: e−jΩT/2 -- phase =−ΩT/2, linear in Ω, corresponding to a pure delay of T/2 (half a sample period), the group delay of the hold.

- Magnitude term: T sincΩT/2π -- a sinc-shaped rolloff that suppresses high frequencies (the familiar "sinc droop" of a DAC).

### c) Normalized magnitude in dB

Let ω=ΩT. Then HZOH/T=sincω/2π.

At ω=0.5π:

ω2π=0.25,  sinc0.25=sin0.25π0.25π=0.70710.7854=0.9003

20log100.9003≈−0.91 dB

At ω=π:

ω2π=0.5,  sinc0.5=2π=0.6366

20log100.6366≈−3.92 dB

(This is the well-known ZOH droop of about −3.9 dB at the Nyquist frequency.)

## 3. Uniform Quantization and SQNR

Range: ≈−1 V to +1 V, full-scale span =2 V.

### a) Step size for B=8

Δ=22B=2256=1128=0.0078125 V

### b) Error variance and RMS error

σe2=Δ212=0.0078125212=5.086×10−6 V2

σe=σe2=Δ12=0.002255 V ≈2.26 mV

### c) SQNR for 8-bit full-scale sine

Standard result: for a full-scale sinusoid quantized with B bits (uniform-error model),

SQNR(dB)=6.02 B+1.76

SQNR8=6.028+1.76=48.16+1.76=49.92 dB

### d) SQNR for 16-bit full-scale sine

SQNR16=6.0216+1.76=96.32+1.76=98.08 dB

### e) Reconciling "96 dB" vs "98 dB" for 16-bit audio

These numbers are not contradictory because they answer different questions:

- "~96 dB" is the common rule-of-thumb figure 6.02 B≈96.3 dB. It represents the dynamic range of the quantizer in a loose sense -- essentially 20log102B, the ratio of the full-scale range to the quantization step, without including the +1.76 dB correction factor. It's often quoted as a quick "6 dB per bit" estimate and is sometimes meant as a conservative, generic dynamic-range spec rather than a precise SQNR for one specific signal.

- "~98 dB" is the exact SQNR formula result for the specific case of a full-scale sinusoidal test signal, which includes the +1.76 dB term that comes from the ratio of a sine wave's mean-square power to its peak value (Vrms2=Vpk2/2).

So the 96 dB figure is a rounded/approximate dynamic-range statement (or omits the sine-specific correction), while 98 dB is the precise theoretical SQNR for a full-scale sine tone. Both are legitimate -- they simply correspond to slightly different definitions/assumptions (generic dynamic range vs. exact full-scale-sinusoid SQNR), and real-world 16-bit audio numbers can also be affected by dither, noise shaping, and measurement weighting, which further explains why quoted "16-bit" figures vary in the literature.

## 4. Finite Sequence z-Transform

Given: xn=2δn+1−δn+3δn−2

### z-Transform

Xz=n​xnz−n=2z1−1+3z−2

Xz=2z−1+3z−2

### ROC

The sequence has both a positive-time term (2z1, i.e. nonzero at n=−1) and negative-time-index terms extending to n=2 (3z−2). Because there is a term 2z that diverges as z→∞, and a term 3z−2 that diverges as z→0, the ROC is the entire z-plane except the origin and infinity:

ROC: 0<z<∞

### Does the DTFT exist?

Since xn has finite support (only 3 nonzero samples), the sum defining Xejω is a finite sum and converges for every ω. The unit circle z=1 lies within the ROC, so yes, the DTFT exists (it is simply Xejω=2ejω−1+3e−j2ω).

## 5. Right-Sided Exponential and Convergence

Given: xn=1.2nun

### z-Transform via geometric series

Xz=n=0∞1.2nz−n=n=0∞1.2 z−1n

This geometric series converges when 1.2 z−1<1, i.e. z>1.2, giving:

Xz=11−1.2 z−1,  ROC: z>1.2

### Pole

Pole at z=1.2

### Does the DTFT exist?

No. The ROC z>1.2 does not include the unit circle z=1 (since 1.2>1). Equivalently, 1.2nun grows without bound and is not absolutely (or even square) summable, so the DTFT does not converge.

## 6. Same Algebra, Different ROC

Given: Xz=11−0.75 z−1

### a) ROC: z>0.75

This ROC is the exterior of the pole, associated with a right-sided (causal) sequence:

xn=0.75nun

### b) ROC: z<0.75

This ROC is the interior of the pole, associated with a left-sided (anticausal) sequence:

xn=−0.75nu−n−1

### Why the same rational expression gives two different answers

The algebraic expression Xz alone only fixes the poles/zeros; the ROC determines which side of each pole the power series is expanded on (causal vs. anticausal), so specifying the ROC is essential to uniquely invert a rational Xz.

## 7. Partial Fractions with Two ROCs

Given: Xz=11−0.5z−11−0.8z−1

### Partial fraction expansion

Write Xz=A1−0.5z−1+B1−0.8z−1.

Substituting x=z−1: 1=A1−0.8x+B1−0.5x

- At x=1/0.5=2:  1=A1−1.6=−0.6A ⇒ A=−53

- At x=1/0.8=1.25:  1=B1−0.625=0.375B ⇒ B=83

(Check: A+B=−5/3+8/3=1 âœ“, matching Xz|x=0=1.)

Xz=−5/31−0.5z−1+8/31−0.8z−1

Poles at z=0.5 and z=0.8.

### a) ROC: z>0.8 (outside both poles -- causal/stable-looking, right-sided)

Both terms are inverted as right-sided sequences:

xn=−530.5nun+830.8nun

### b) ROC: 0.5<z<0.8 (between the two poles)

This ROC lies outside the pole at z=0.5 (so that term is right-sided) but inside the pole at z=0.8 (so that term is left-sided/anticausal):

xn=−530.5nun − 830.8nu−n−1

This is a genuinely two-sided sequence -- decaying for n→+∞ (from the 0.5-pole term) and also decaying for n→−∞ (from the 0.8-pole term), which is exactly what an annular ROC between two poles signals.