ECEN 4/5632  Theory and Application of Digital Filters  Fall 2026

Exit Ticket Questions for Week 1

1. Why does h[n] completely characterize an LTI system?

2. Why can an LTI system change the amplitude and phase of a sinusoid, but not its frequency?

Good answers:

1. Any discrete-time signal can be written as a linear combination of x[k] times the basis function δ[n-k]. By linearity, the response to a scaled impulse is scaled and by time-invariance the response to δ[n-k] is h[n-k]. Therefore, again by linearity, yn=kxkh[n-k]=xn*h[n].

Look for both linearity and time-invariance in the answer.

2. Complex exponentials are eigenfunctions of LTI systems: Input xn=ejω0n produces output yn=Hejω0ejω0n (due to convolution which depends on linearity and time-invariance). The multiplier H(ejω0) can change magnitude and phase, but the factor ejω0n remains at the same frequency.

Look for eigenfunctions, eigenvalues and convolution (which requires linearity and time-invariance) in the answer.

Answers:

Mediocre answer: Because an LTI system is defined by its impulse response (circular response)

Good answer: Because every input can be decomposed into scaled and shifted impulses, and linearity and time-invariance tell us that the output is the corresponding sum of scaled and shifted copies of h[n].

2. Complex exponentials are eigenfunctions of LTI systems: xn=ejω0n produces yn=khkejω0(n-k)=ejω0nkhke-jω0k=ejω0nH(ejω0). The eigenvalue H(ejω0) is a constant with magnitude and phase and only the factor ejω0n, which is the same for the input and the output of the LTI system (eigenfunction), determines frequency.

Weak answer: Because LTI systems preserve frequency. (true but circular.)

Good answer: Due to the convolution equation yn=khkx[n-k], the output y[n] of an LTI system in response to a rotating phasor xn=ejω0n is a linear combination of scaled and time shifted rotating phasors hkejω0(k-n) of the same frequency as the input phasor.

The core reason: sinusoids are eigenfunctions of LTI systems.

Linearity is what lets you factor ejωn out and lets superposition apply — the output at frequency ω depends only on the input at frequency ω, not on cross-terms mixing different frequencies together.

Time-invariance is what makes Hejω a constant instead of a function of n. If the system's parameters changed with time, h would depend on both n and k, and the ejωn term wouldn't factor cleanly out of the integral

A good exit ticket answer should be short, correct, to the point and demonstrate a well-reasoned understanding of the question being asked. Exit ticket answers will be graded on a scale of “low” (1 point), “medium” (2 points), or “high” (3 points).

Use LaTeX style for math expressions, e.g., \delta[n] for a discrete-time impulse.