ECEN 4/5632         Theory and Application of Digital Filters          Fall 2026

9-5-26                                                                                                                                               P. Mathys

Problem Set 3: Solution

1. Ideal sinc reconstruction

Let the discrete-time sequence be x[n]=δ[n]+2δ[n-1], obtained from a valid bandlimited sampling process with sample period T.

a)  Write the ideal continuous-time reconstructed signal xr(t) using normalized sinc(u)=sin(πu)/(πu).

b)  Verify xr(0)=1 and xr(T)=2.

c)  Compute xr(T/2).

2. Zero-order hold frequency response

A DAC uses a zero-order hold hZOH(t)=u(t)-u(t-T).

a)  Derive HZOH(jΩ) by direct integration.

b)  Separate the result into a linear-phase term and a sinc-like magnitude term.

c)  Compute the normalized magnitude |HZOH|/T in dB at ω=ΩT=0.5π and at ω=π.

3. Uniform quantization and SQNR

A uniform B-bit quantizer spans approximately −1 V to +1 V. Assume the high-resolution uniform-error model when requested.

a)  For B=8, find the quantization step Δ.

b)  Find the modeled error variance σ_e²=Δ²/12 and RMS quantization error.

c)  Estimate the SQNR of a full-scale 8-bit sine.

d)  Estimate the full-scale sine SQNR for 16 bits.

e)  Explain why “16-bit audio is about 96 dB” and “16-bit full-scale sine SQNR is about 98 dB” are not necessarily contradictory

4. Finite sequence z-transform

Let x[n] = 2δ[n+1] - δ[n] + 3δ[n-2]. Find X(z), state the ROC, and determine whether the DTFT exists.

5. Right-sided exponential and convergence

For x[n]=(1.2)ⁿ u[n], derive X(z) from the geometric series, state the ROC, identify the pole, and determine whether its DTFT exists.

6. Same algebra, different ROC

Find the inverse z-transform of X(z)=1/(1-0.75z⁻¹) for

a) |z|>0.75 and

b) |z|<0.75.

Explain in one sentence why the two answers can have the same rational expression.

7. Partial fractions with two ROCs (ECEN 5632 Problem)

Let X(z)=1/[(1-0.5z⁻¹)(1-0.8z⁻¹)]. Use partial fractions and find x[n] for

a) |z|>0.8 and

b) 0.5<|z|<0.8.