# PS04_Solution_ClaudeAI

## ClaudeAI solution

# ECEN 4/5632 — Problem Set 4 Solutions

## Problem 1: Difference Equation to H(z)

Given: y[n] − 1.4y[n−1] + 0.45y[n−2] = x[n] + x[n−1]

### a) Find H(z)

Taking the Z-transform of both sides (assuming zero initial conditions):

Y(z)[1 − 1.4z⁻¹ + 0.45z⁻²] = X(z)[1 + z⁻¹]

H(z)=1+z−11−1.4z−1+0.45z−2

### b) Zeros and poles

Multiply numerator and denominator by z²:

H(z)=z2+zz2−1.4z+0.45=z(z+1)z2−1.4z+0.45

Zeros: z = 0 and z = −1

Poles: solve z² − 1.4z + 0.45 = 0

z = [1.4 ± √(1.96 − 1.8)]/2 = [1.4 ± 0.4]/2

Poles: z = 0.9 and z = 0.5

### c) ROC for the causal system

For a causal system, the ROC is the region outside the pole of largest magnitude:

ROC: |z| > 0.9

### d) BIBO stability

A causal LTI system is BIBO stable iff the ROC includes the unit circle. Since both poles (0.9 and 0.5) lie strictly inside the unit circle, the causal ROC |z| > 0.9 contains |z| = 1.

Yes — the system is BIBO stable.

## Problem 2: Geometric Frequency-Response Reasoning

Given: H(z) = (1 + z⁻¹)/(1 − 0.8z⁻¹)

### a) Poles and zeros

Writing H(z) = (z + 1)/(z − 0.8):

- Zero: z = −1

- Pole: z = 0.8

### b) Geometric evaluation

For z = e^{jω} on the unit circle:

|H(ejω)|=|ejω−(−1)||ejω−0.8|=distance to zerodistance to pole

At ω = 0 (z = 1):

- Distance to zero at −1: |1 − (−1)| = 2 (long vector — evaluation point is far from the zero)

- Distance to pole at 0.8: |1 − 0.8| = 0.2 (very short vector — evaluation point is right next to the pole)

- |H(e^{j0})| = 2 / 0.2 = 10

At ω = π (z = −1):

- Distance to zero at −1: |−1 − (−1)| = 0 (the evaluation point sits on top of the zero)

- Distance to pole at 0.8: |−1 − 0.8| = 1.8

- |H(e^{jπ})| = 0 / 1.8 = 0

Sketch description: Draw the unit circle with the pole marked as an × at z = 0.8 (on the positive real axis, inside the circle) and the zero marked as an ○ at z = −1 (on the circle itself, at the leftmost point). At ω = 0, the point z = 1 is close to the pole and far from the zero → short pole-vector, long zero-vector → large gain. At ω = π, the point z = −1 coincides exactly with the zero → zero-vector length is 0 → the response is completely nulled.

### c) Conclusion

The gain is large near ω = 0 and drops to exactly zero at ω = π. This is a lowpass filter: it passes low frequencies (and even boosts DC) while completely rejecting the Nyquist frequency.

## Problem 3: 600 Hz Notch Filter Design (f_s = 9000 Hz)

### a) Pole/zero locations

ω0=2πf0fs=2π(600)9000=2π15≈0.4189 rad≈24∘

Zeros (on the unit circle, radius 1, at ±ω₀):

z=e±jω0=cos(24∘)±jsin(24∘)≈0.9135±j 0.4067

Poles (radius r = 0.96, same angle):

z=0.96 e±jω0≈0.8770±j 0.3904

This gives the standard second-order notch transfer function:

H(z)=1−2cos(ω0)z−1+z−21−2rcos(ω0)z−1+r2z−2

### b) Effect of increasing r from 0.96 to 0.99

Moving the poles closer to the unit circle (closer to the zeros) narrows the notch bandwidth, making the null much sharper/more selective around exactly 600 Hz while leaving frequencies just outside the notch nearly unaffected. The trade-off: the impulse response rings longer (slower pole decay, since the pole radius is closer to 1), and the filter becomes more sensitive to coefficient quantization/rounding errors — a small perturbation in r or ω₀ has a larger relative effect on the notch shape when r is very close to 1.

## Problem 4: Ideal Lowpass FIR Filter

### a) Inverse DTFT

hd[n]=12π−ωcωcejωn dω=sin(ωcn)πn, n≠0

hd[0]=ωcπ

(This can also be written as hd[n]=ωcπ sinc​ωcnπ using the normalized sinc, sinc(x) = sin(πx)/(πx).)

### b) Sketch for ω_c = π/3, n ∈ [−8, 8]

hd[0]=π/3π=13≈0.3333

Selected values (h_d is even, h_d[−n] = h_d[n]):

| n | 0 | ±1 | ±2 | ±3 | ±4 | ±5 | ±6 | ±7 | ±8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| h_d[n] | 0.3333 | 0.2757 | 0.1378 | 0.0000 | −0.0689 | −0.0551 | 0.0000 | 0.0394 | 0.0345 |

Sketch description: A symmetric, damped-oscillatory "sinc" shape centered at n = 0. Peak of 1/3 at n = 0, decaying envelope ~1/(πn), with exact zero crossings at every third sample (n = ±3, ±6, ...) because sin(nπ/3) = 0 there. The tails extend (in principle) to n = ±∞, oscillating in sign.

### c) Why this can't be an exact real-time FIR filter

h_d[n] is two-sided (nonzero for n < 0, so it's non-causal) and has infinite duration (it never exactly reaches zero for finite n). A real-time FIR filter must have a finite number of taps and can only use present and past inputs (causal). Since h_d[n] is both infinite in length and non-causal, it cannot be implemented exactly — it must be truncated (finite length) and shifted (to be causal), which is the origin of the windowing/Gibbs phenomenon discussed in Problem 5.

### d) Causal shift for N = 61

For an odd length N, truncate h_d[n] over n ∈ [−(N−1)/2, (N−1)/2] = [−30, 30], then shift right by (N−1)/2 = 30 samples:

h[n]=hd[n−30], n=0,1,…,60

Constant group delay = (N − 1)/2 = 30 samples.

## Problem 5: Window Tradeoff (51-tap filters)

### a) Narrower transition band

Filter A (rectangular window) has the narrower transition band — the rectangular window has the narrowest main lobe of common windows for a given length.

### b) Lower stopband sidelobes

Filter B (Hamming window) has much lower stopband sidelobes, because the Hamming window's smooth tapering greatly suppresses sidelobe energy compared to the abrupt rectangular truncation.

### c) Approximate first-sidelobe levels

- Rectangular window: first sidelobe ≈ −13 dB relative to the main lobe peak

- Hamming window: first sidelobe ≈ −41 to −43 dB relative to the main lobe peak (minimum stopband attenuation typically quoted around 53 dB)

### d) Why longer rectangular windows don't remove Gibbs overshoot

Increasing the rectangular window length N narrows the transition band (compresses the ripple pattern closer to the discontinuity in frequency), but it does not reduce the height of the overshoot. This is the Gibbs phenomenon: near a discontinuity, truncating a Fourier series produces a fixed ~9% overshoot of the jump size regardless of how many terms are kept. Increasing N only moves the ripples closer to the discontinuity and increases their density — it does not make the peak overshoot amplitude shrink. To reduce the overshoot itself you need a different (tapered) window, such as Hamming, not just a longer rectangular one.

## Problem 6: Symmetry and Linear Phase

Given: real FIR filter of length N with h[n] = h[N − 1 − n], n = 0, ..., N − 1.

### a) Derivation

Let M = (N − 1)/2 (the center of symmetry). Factor out the linear-phase term:

H(ejω)=n=0N−1h[n]e−jωn=e−jωMn=0N−1h[n] e−jω(n−M)

Substitute m = n − M, so the sum runs over m = −M, ..., M (symmetric interval, since N − 1 − M = M). Define g[m] = h[m + M]. Because h[n] = h[N − 1 − n], we get g[m] = g[−m]: the shifted sequence is even.

m=−MMg[m]e−jωm=g[0]+m=1Mg[m]e−jωm+ejωm=g[0]+2m=1Mg[m]cos(ωm)

The right-hand side is manifestly real-valued — call it A(ω). Therefore:

H(ejω)=e−jω(N−1)/2 A(ω),  A(ω) real

which is exactly the required form.

### b) Group delay and relevance to ML feature extraction

The phase is θ(ω)=−ω(N−1)/2 wherever A(ω) ≠ 0 (A(ω) can go negative, contributing an extra constant π jump, but the linear term is unaffected). The group delay is:

τg(ω)=−dθ(ω)dω=N−12 (constant, independent of ω)

Why this matters for ML feature extraction: a constant group delay means every frequency component of the signal is delayed by exactly the same number of samples — there is no phase distortion, so the filtered waveform is simply a delayed, undistorted copy of the shape that would result from an ideal (zero-delay) filter. In a feature-extraction pipeline (e.g., a filterbank producing multiple sub-band signals, as in MFCC-style audio features), this guarantees that transients, onsets, and relative timing relationships stay aligned across all filter channels — the fixed, known delay (N−1)/2 can simply be subtracted out. Nonlinear-phase filters would smear or shift different frequency components by different amounts, corrupting timing-sensitive features and complicating alignment across channels.

## Problem 7: Pole Radius and Time-Domain Decay

For a causal system with a complex-conjugate pole pair p=re±jθ, a partial-fraction expansion of H(z) contains terms of the form:

A pnu[n]+A* p*nu[n]=2|A| rncos(nθ+ϕ) u[n]

Why the decay is r^n: each pole contributes an impulse-response term proportional to pn=rnejnθ. The magnitude of this term is |p|n=rn exactly, since |p| = r. The angle θ only produces oscillation (a damped sinusoid); it's the magnitude r that controls the decay envelope.

Why r → 1 increases ringing duration: the "time constant" of the decay behaves like τ≈−1/lnr. As r → 1⁻, ln r → 0⁻, so τ → ∞ — the envelope r^n decays more and more slowly, meaning the impulse response keeps oscillating (ringing) for many more samples before becoming negligible.

Why r → 1 sharpens the frequency resonance: geometrically, |H(ejω)| near the pole's angle is inversely proportional to the distance from the point ejω on the unit circle to the pole. That distance reaches its minimum value of (1 − r) exactly at ω = θ. As r → 1, this minimum distance (1 − r) → 0, so the reciprocal-distance factor blows up right at ω = θ, producing a tall, narrow resonance peak — i.e., a high-Q (high-quality-factor) filter.

Connecting the two effects: both are governed by the same distance (1 − r) of the pole from the unit circle. A pole close to the unit circle is simultaneously (1) close in distance to the point e^{jθ}, producing a sharp/narrow frequency peak, and (2) close to magnitude 1, producing very slow amplitude decay in time. This is a time–bandwidth duality: narrow bandwidth (sharp resonance) and long ringing time are two manifestations of the same underlying geometric fact — the pole sitting near the unit circle. Roughly, the resonance's 3-dB bandwidth Δω≈2(1−r) and the ringing time constant τ≈1/(1−r), so Δω⋅τ≈ constant: you cannot make the resonance arbitrarily sharp without paying for it in longer time-domain ringing, and vice versa.