ECEN 4/5632         Theory and Application of Digital Filters          Fall 2026

10-10-26                                                                                                                                           P. Mathys

Problem Set 8

DFT/FFT Conventions:

XNk=n=0N-1xne-j2πkn/N

xn=1Nk=0N-1XNkej2πknN

Use Lx for the number of acquired samples in x[n] and N for the DFT/FFT length. If N>Lx, append zeros after the Lx samples (zero-padding). All spectral experiments in this problem set use complex sinusoids to isolate positive frequency components: normalize their magnitudes by Lx, not N.

1. Periodic representations of DFT/FFT

x0:3=1,2,0,-1

(a) Compute the four point DFT by hand. Show the arithmetic and intermediate terms. Use an FFT only to verify your result.

(b) Extend the DFT formula to any integer k (i.e., including k outside the 0…N-1 range). Show that Xk+4 equals X[k]. What frequencies do bins k=3 and k=-1 represent relative to the sampling frequency fs?

(c) Evaluate the inverse DFT expression at n=-1, 4, 5 and 6. Show that this expression repeats every four samples. Distinguish this periodic representation from the finite sequence x[n] taken to be zero outside n=0…3.

(d) Does an FFT change either of these periodicity properties? Explain.

2. A delay that wraps around

Let the eight sample record x8[n] be the sequence below and define its eight point DFT to be X8[k].

x80:7=[1, 2, 3, 4, 0, 0, 0, 0]

Y8k=X8ke-j2π6k/8

(a) Find the eight sample inverse DFT of Y8[k]. Derive the circular shift relation and identify exactly which samples wrap around.

(b) Instead interpret the original signal as zero outside its nonzero support, samples 0 through 3. Delay it by six samples on the infinite time axis. List its first eight samples. Explain why this differs from part (a), although both calculations are often called a delay.

(c) Append zeros to create a 16 sample record, compute the 16-point DFT, and apply the phase ramp with denominator 16. List the complete output. What is the smallest transform length that avoids wrapping any non-zero delayed sample for this signal? Explain using its support.

(d) Does padding eliminate DFT periodicity, or does it change the period and provide space before non-zero samples would wrap? Explain.

3. Circular and linear convolution

x=1 ,2 ,3, 4,             h=[1,-1, 1]

(a) Compute the full linear convolution by hand.

(b) Compute IFFT(FFT(x,N) times FFT(h,N)) for N=4 and N=5. Predict both results by folding the full linear convolution before running Python. Show the folding arithmetic.

(c) Find the minimum N that permits the FFT product to recover the full linear convolution without time aliasing. Verify at that length and at N=8, including any trailing zero padding.

(d) Explain why using a longer FFT can fix circular convolution aliasing, even though a longer FFT does not necessarily separate two close tones. Identify what the padding accomplishes in each setting.

4. Boundaries and  spectral leakage

Use fs=1024 samples per second and acquire Lx=64 complex samples. Consider two signals, each separately:

xfn=ej2πfn/fs ,            0≤n<64 .

Use f=128 Hz for the first signal and f=136 Hz for the second signal. Use a rectangular observation window unless part (d) specifies otherwise.

(a) Derive the finite record DTFT as a geometric sum. Express its magnitude as a ratio of sine functions, including the limiting value at the tone frequency. State where the 64-point DFT samples this DTFT.

(b) Compute the number of cycles of the two signals in each 64 sample observation interval. Evaluate the underlying complex sinusoid at n=64 and compare with x[0]. Which sinusoid matches the continuation implied by repeating the record periodically? Note that the matching condition is x64=x[0] for the underlying tone, not equality of the last stored sample x[63] and x[0]. Explain why.

(c) Plot DFT magnitudes for N=64 and N=1024 on the same frequency axes for each tone. Use magnitude in dB relative to 1, normalized by Lx, and a display floor of -60 dB. Explain why the unpadded 128 Hz case has only one nonzero DFT bin, yet its zero-padded plot reveals sidelobes between the original DFT sample frequencies. Did padding create these lobes?

(d) For the 136 Hz tone compare rectangular and periodic (as opposed to symmetric) Hann windows on a dense (frequency) grid. Normalize by each window sum. Explain the changes in sidelobes and main lobe width. Can tapering generally remove all leakage while retaining a rectangular window’s resolving power?

5. Zero padding and frequency plot detail (ECEN 5632 Problem)

Acquire a single complex-valued sinusoid with amplitude 1, fs=2048 samples per second, f0=403.3 Hz and Lx=128 samples. Keep these 128 measured samples fixed throughout the problem.

xn=ej2πf0n/fs ,               0≤n<128 .

(a) Compute spectra for N=128, 512 and 4096. Overlay or compare plots over 350 to 450 Hz using magnitude divided by 128. Make a table of N, frequency grid spacing, largest bin frequency in the 350 to 450 Hz interval, and its amplitude. Explain the improvement in locating the sampled peak.

(b) Write the M-point DFT of the zero padded record. Prove that it samples the same finite record DTFT at a denser set of frequencies. If M=qLx for integer q, identify exactly which padded DFT bins equal the original Lx-point DFT bins. Verify the result for q=4.

(c) Find the first spectral zeros on either side of f0 and the null-to-null main lobe width. Do these frequencies change as N increases? Explain the distinction between plot grid spacing and main lobe width.

(d) Use the Lx-point inverse DFT to show that every M-point zero padded value can be calculated from the original LxDFT values. Give an explicit interpolation sum. Explain why this establishes that padding supplies no additional measured information.

(e) Explain why normalization by N would falsely suggest that the signal amplitude decreases with padding. Explain why improved single tone peak localization does not establish improved two tone separation.

Note: Use N=128, 512 and 4096 only as computational and display choices; do not acquire additional signal samples in this problem.