# One week DSP project

Read the assignment before starting. Implement student_dsp.py; all four functions currently raise NotImplementedError. Run from the folder containing this notebook. The clean references are for evaluation only.

from pathlib import Path
import numpy as np
import matplotlib.pyplot as plt
from scipy import signal
from common import FS, fixtures, read_wav, relerr, snr, receive
import student_dsp as dsp
a = fixtures(Path("signals"))
# Restart the kernel after editing student_dsp.py, or reload the module.

## A1 STFT and uncertainty

Implement analysis, then plot the probe at L=256, 1024, 4096 with 75% overlap. Use identical display limits and explain the tone/transient tradeoff. Repeat L=1024 with N=4096. Keep magnitude or relative power labels separate from calibrated PSD labels.

L = 1024
X, meta = dsp.analysis(a["probe"], L=L, hop=L//4)
# TODO: label centered frame times using the leading padding offset.
# TODO: plot nonnegative frequencies for the real audio.

## A2 Window leakage

Use the four assigned windows. Measure window response main lobe and sidelobes separately from the two tone spectrum. Correct tone amplitudes using the window sum.

n = np.arange(1024)
tone = .65*np.cos(2*np.pi*1003*n/FS) + .015*np.cos(2*np.pi*1140*n/FS)
# TODO: rectangular, Hann, Hamming, Blackman at N=65536.

## B1 Overlap and synthesis

Implement synthesis and sweep the five overlaps. A Hann NOLA failure at zero overlap is an expected control. Use a rectangular window to show successful zero overlap reconstruction.

X, meta = dsp.analysis(a["probe"], L=1024, hop=256)
y, denominator = dsp.synthesis(X, meta)
assert relerr(a["probe"], y) < 1e-10
# TODO: table of overlaps, frame counts, denominators, runtime and errors.

## B2 Exact FFT overlap add

Implement full linear convolution. Compare against direct convolution, including the filter tail and final partial block.

h = signal.firwin(129, 1200, fs=FS, window="hamming")
y = dsp.ola_filter(a["mixture"], h, B=512)
reference = np.convolve(a["mixture"], h)
assert len(y) == len(reference)
assert relerr(reference, y) < 1e-10
# TODO: repeat for B=128 and 2048.

## C1 Nonstationary jammer

The algorithm receives the mixture and search band only. Estimate and smooth a ridge, multiply complex coefficients by a symmetric tapered mask, and reassemble. Freeze that mask before evaluating clean distortion and jammer attenuation.

y, diagnostics = dsp.remove_jammer(a["mixture"])
print("Input SNR:", snr(a["melody"], a["mixture"]))
print("Output SNR:", snr(a["melody"], y))
# TODO: width/frame-length sweep, stationary control, matched-scale WAVs.

## C2 QPSK phase coherence

Keep I/Q complex, without averaging WAV channels. Use fixed receiver timing. Plot constellations and compare against magnitude-only and independently rotated frames. No phase or gain correction is allowed in the controls.

X, meta = dsp.analysis(a["tx"])
y, denominator = dsp.synthesis(X, meta)
metrics, z = receive(y, a["symbols"], a["bits"], a["pulse"])
print(metrics)
# TODO: magnitude-only and frame random-phase controls; constellation plots.

## Report

Include parameter choices, evidence, explanations, limitations and contributions. Graduate students include the normalization derivation and chirp-spreading interpretation. See the 100 point rubric in the assignment.