# Week 2 easier

Week 2: Sampling Theory. Continuous-Time to Discrete-Time (C/D) conversion, D/C conversion, Nyquist theorem, aliasing, and quantization noise.

Week 2 easier

Can you map out the two 75-minute lectures for Week 2 covering Sampling, Aliasing, and C/D conversion?

Gemini Pro:

Here is the detailed breakdown for the two 75-minute lectures in Week 2.

This week bridges the physical world (continuous-time acoustic waves) and the digital domain (discrete-time arrays). It is vital to emphasize that any mistakes made during sampling (like aliasing) permanently destroy information, which will later ruin the performance of their machine learning models.

## Lecture 3: Continuous-to-Discrete (C/D) Conversion & Aliasing

Objective: Mathematically model the sampling process using impulse trains, derive the frequency-domain spectrum of a sampled signal, and prove the Nyquist-Shannon Sampling Theorem.

| Time Block | Topic & Mathematical Focus | Teaching Strategy & Audio Connection |
| --- | --- | --- |
| 00 - 15 min | The C/D Interface | Motivation: Microphones capture continuous-time voltage xct, but PyTorch only accepts discrete arrays xn. Define the relationship xn=xcnT. |
| 15 - 35 min | Ideal Impulse Train Sampling | Introduce the mathematical model. Multiply xct by an impulse train st=∑δt−nT to create the modulated signal xst. |
| 35 - 55 min | Sampling in the Frequency Domain | Derive the spectrum of xst. Show that multiplication in time is convolution in frequency, resulting in infinite, scaled copies of the baseband spectrum separated by Ωs. |
| 55 - 75 min | Nyquist Theorem & Aliasing | Define the Nyquist rate mathematically: Ωs>2ΩN. Draw what happens on the board when Ωs<2ΩN (the spectral copies overlap). Audio ML connection: Show how aliased high-frequency noise creates false patterns in a spectrogram. |

### Key Equations to Board

- Impulse Train Sampling:

xst=xctn=−∞∞δt−nT=n=−∞∞xcnTδt−nT

- Spectrum of Sampled Signal:

XsjΩ=1Tk=−∞∞XcjΩ−kΩs

- Continuous vs. Discrete Frequency:

ω=ΩT

## Lecture 4: Reconstruction, D/A, and Quantization Noise

Objective: Explain the mathematics of ideal reconstruction (D/C conversion), the realities of physical Digital-to-Analog hardware, and model the effects of bit-depth (quantization) on audio quality.

| Time Block | Topic & Mathematical Focus | Teaching Strategy & Audio Connection |
| --- | --- | --- |
| 00 - 20 min | Ideal D/C Conversion | Explain how to isolate the baseband spectrum using an ideal continuous-time low-pass filter HrjΩ. Derive the time-domain interpolation formula using the sinc function. |
| 20 - 40 min | Practical D/A: Zero-Order Hold | The math of ideal sinc interpolation is non-causal and infinitely long. Introduce the Zero-Order Hold (ZOH) circuit. Show how ZOH introduces a sinc-shaped envelope drop-off in the frequency domain. |
| 40 - 55 min | Quantization & Bit-Depth | Time is discrete, but amplitude must also be discretized. Model quantization as an additive, wide-sense stationary noise source en. Derive the 6.02 dB per bit rule. |
| 55 - 75 min | Live Audio Demo | Take a clean vocal recording (24-bit, 48 kHz). Decimate it without an anti-aliasing filter to hear aliasing (sounds like robotic ringing). Then reduce it to 8-bit and 4-bit to hear quantization noise (sounds like loud white noise/fuzz). |

### Key Equations to Board

- Ideal Bandlimited Interpolation:

xrt=n=−∞∞xnsinπt−nT/Tπt−nT/T

- Quantization Noise Model:

xqn=xn+en

- Signal-to-Quantization-Noise Ratio (SQNR):

SQNR≈6.02B+1.76 dB

(where B is the number of bits).

Instructor Tip: When discussing quantization, explicitly connect B (bit depth) to data types they will use in PyTorch. For instance, explain why raw audio is usually 16-bit integer (giving ∼96 dB of dynamic range), but neural networks require casting these to 32-bit floats.

Prompt: Can you create slides and python code for these two 75 minute classes of a DSP and ML course for a combined senior and first year Electrical and Computer Engineering class? The DSP textbook is Discrete-Time Signal Processing by Oppenheim and Schafer. Add some homework problems with solutions and a weekly quiz with solutions as well.

Chat GPT:

Done. I created the complete Week 2 DSP + ML teaching package for the two 75-minute classes.

### Complete package

Download the complete Week 2 package

### Individual files

- Lecture 3 — Continuous-to-Discrete Conversion & Aliasing — 22 slides with speaker notes and timing guidance.

- Lecture 4 — Reconstruction, D/A, and Quantization Noise — 24 slides with speaker notes and the live-demo sequence.

- Week 2 Jupyter Notebook — executed notebook with NumPy, SciPy, Matplotlib, PyTorch, spectrograms, and playable audio.

- Standalone Python Code

- Homework + Weekly Quiz + Complete Solutions

### Audio files for the Lecture 4 demo

- Clean 48 kHz reference

- Naive 48 → 12 kHz decimation with aliasing

- Anti-aliased 48 → 12 kHz decimation

- 8-bit quantized audio

- 4-bit quantized audio

A few teaching choices I built in deliberately:

Lecture 3 treats aliasing as a non-uniqueness problem, not merely as vague “distortion.” Students see mathematically that a 2 kHz and 10 kHz cosine sampled at 12 kHz generate identical samples. That makes the argument that ML cannot magically recover lost information particularly strong.

The spectral-replica derivation goes all the way through

SjΩ=2πTk​δΩ−kΩs

to

XsjΩ=1Tk​XcjΩ−kΩs,

so the Nyquist condition follows geometrically rather than appearing as a rule to memorize.

Lecture 4 derives both sinc reconstruction and the ZOH response. The ZOH section shows why

HZOHjΩ=Te−jΩT/2sinΩT/2ΩT/2

produces the familiar (-3.92) dB droop at Nyquist.

I also included a distinction that is often confusing in audio courses: the roughly 96 dB “6 dB/bit” figure for 16 bits versus the 98.08 dB full-scale-sine SQNR obtained from (6.02B+1.76).

The PyTorch material explicitly emphasizes that converting int16 PCM to float32 changes the computational representation but does not recover quantization precision or undo aliasing.

The homework contains 8 core problems plus a graduate stretch problem, and the weekly quiz is a 20-point, approximately 15-minute assessment with a grading guide. The notebook was executed successfully, both slide decks were rendered and checked for overflow, and the homework/quiz document was rendered page-by-page for layout verification.

# Week 2, Lecture 3, LaTeX math

Here is the lecture content converted with properly formatted LaTeX for all the mathematical expressions, organized for clear readability:

This lecture addresses a fundamental question in signal acquisition:

When do samples contain enough information to uniquely represent the original analog signal?

The core mathematical progression flows as follows:

xct→impulse-train sampling→XsjΩ→spectral replicas→Nyquist condition→aliasing if replicas overlap

Framing sampling as an information-preservation problem rather than merely taking measurements every T seconds highlights a critical machine learning consequence: if aliasing introduces false spectral structure before the data reaches a model, the model cannot distinguish that artificial structure from authentic acoustic features.

## 1. The C/D Interface: What Actually Changes When We Sample?

We begin with a continuous-time signal xct. For audio applications, this represents continuous physical phenomena such as microphone voltage.

Sampling every T seconds yields:

xn=xcnT

where

fs=1T

Key variables:

- t: Continuous time variable (seconds)

- n: Discrete integer sample index

- T: Sampling period (seconds/sample)

- fs: Sampling rate (samples/second)

Sampling discretizes time, whereas quantization discretizes amplitude.

### Example 1

At fs=48 kHz, what physical time corresponds to sample n=2400?

t=240048000=0.05 s⟹50 ms

### Example 2

Does ideal sampling change amplitude values?

- Answer: No. Ideal sampling merely evaluates the continuous-time function at discrete points. Amplitude restriction occurs later during quantization.

## 2. Continuous Frequency vs. Discrete Frequency

A continuous-time sinusoid is represented as:

xct=cosΩ0t

where Ω0 is continuous angular frequency in rad/s. Sampling yields:

xn=cosΩ0nT

Defining the normalized discrete-time frequency ω0:

ω0=Ω0T

gives:

xn=cosω0n

Here, ω carries units of rad/sample and is unique only modulo 2π. Because Ω=2πf, the relationship simplifies to:

ω=2πffs

## 3. Normalized Frequency Mapping

At fs=12 kHz, the normalized discrete-time frequency corresponding to a 3 kHz sinusoid is:

ω=2π312=π2

For a 15 kHz sinusoid at the same sample rate:

ω=2π1512=5π2

Evaluating modulo 2π:

5π2≡π2

Thus, 3 kHz and 15 kHz map to the exact same discrete-time complex exponential when sampled at 12 kHz.

## 4. The Impulse Train as an Analytical Tool

Ideal impulse-train sampling models physical sampling mathematically. Defining the periodic impulse train:

st=n=−∞∞δt−nT

The sampled continuous-time signal xst is:

xst=xctst=xctn=−∞∞δt−nT

Applying the sifting property xctδt−nT=xcnTδt−nT:

xst=n=−∞∞xcnTδt−nT

Each impulse location marks a sample instant, with the discrete sequence xn embedded as the weights of the impulses.

## 5. Sample Weight Extraction

Consider xct=cos2π⋅1000t sampled at fs=8000 Hz (T=18000 s).

The impulse weights evaluate to:

xcnT=cos2π⋅1000n8000=cosπn4

Yielding the impulse-train representation:

xst=n=−∞∞cosπn4δt−nT

The underlying discrete sequence is extracted directly from these weights: xn=cosπn4.

## 6. Frequency Representation of the Impulse Train

A periodic impulse train in the time domain transforms into an impulse train in the frequency domain. For st=n=−∞∞δt−nT, its Continuous-Time Fourier Transform (CTFT) is:

SjΩ=2πTk=−∞∞δΩ−kΩs

where

Ωs=2πT=2πfs

This relationship (T↔Ωs=2πT) causes spectral replication in the frequency domain.

## 7. Derivation of Spectral Replicas

Because multiplication in time corresponds to convolution in frequency:

XsjΩ=12πXcjΩ*SjΩ

Substituting SjΩ:

XsjΩ=12πXcjΩ*2πTk=−∞∞δΩ−kΩs

Simplifying:

XsjΩ=1TXcjΩ*k=−∞∞δΩ−kΩs

Since convolution with a shifted Dirac impulse shifts the spectrum XcjΩ*δΩ−kΩs=XcjΩ−kΩs:

XsjΩ=1Tk=−∞∞XcjΩ−kΩs

Sampling replicates the original continuous spectrum XcjΩ at integer multiples of the sampling frequency Ωs, scaled by 1T.

## 8. Spectral Overlay Visualization

Assuming XcjΩ is bandlimited to ΩM (XcjΩ=0 for |​Ω|​>ΩM):

______
            /      \
-----------/        \-----------
        -ΩM     0      ΩM

Sampling produces infinitely many shifted copies centered at 0,±Ωs,±2Ωs,…:

copy             baseband              copy
       /\                 /\                  /\
------/  \---------------/  \----------------/  \------
   -Ωs                 0                    Ωs

As long as neighboring spectral replicas do not overlap, the baseband signal remains intact.

## 9. Derivation of the Nyquist Condition

The baseband spectrum occupies the range −ΩM≤Ω≤ΩM (total width 2ΩM).

The adjacent positive replica is centered at Ωs, extending down to its left edge at Ωs−ΩM. To prevent spectral overlap:

Ωs−ΩM>ΩM⟹Ωs>2ΩM

Expressed in Hz:

fs>2fM

When this condition holds, an ideal low-pass filter with cutoff frequency Ωc=Ωs2 isolates the original spectrum XcjΩ perfectly.

## 10. Nyquist Frequency vs. Nyquist Rate

- Nyquist Frequency (fN): A property of the system. Given sampling rate fs:

fN=fs2

(e.g., fs=48 kHz⟹fN=24 kHz)

- Nyquist Rate: A property of the signal. Given maximum signal frequency fM:

Nyquist Rate=2fM

(e.g., fM=18 kHz⟹Nyquist Rate=36 kHz)

## 11. Practical Guard Bands

While theoretical derivations state fs≥2fM, physical systems cannot construct ideal rectangular brick-wall filters. Practical anti-aliasing filters require a finite transition band:

fM<fs2

Physical analog front-ends need operating margin between the maximum signal frequency and fN.

## 12. Defining Aliasing

Aliasing is non-uniqueness.

When spectral replicas overlap, higher-frequency components fold into lower frequencies. Aliasing is non-invertible distortion; it creates deterministic, false sinusoidal components in the passband that are identical to real lower-frequency signals.

## 13. Indistinguishable Discrete-Time Sinusoids

Consider fs=12 kHz comparing x1t=cos2π⋅2000t and x2t=cos2π⋅10000t.

Sampling x1t:

x1n=cos2π200012000n=cosπn3

Sampling x2t:

x2n=cos2π1000012000n=cos5πn3

Using periodicity 5π3=2π−π3 and cosine symmetry:

cos5πn3=cos2πn−πn3=cos−πn3=cosπn3

x1n=x2n

Both continuous-time signals produce identical digital samples.

## 14. Irreversibility

Once sampled into xn=cosπn3, all historic context regarding whether the source frequency was 2 kHz or 10 kHz is lost. The sample sequence contains no metadata identifying its original continuous-time frequency.

## 15. The Folded Frequency Formula

For a continuous sinusoid of frequency f sampled at fs, the observed alias frequency falias in the baseband 0≤falias≤fs2 satisfies:

falias=|​f−kfs|​ for k∈Z

For fs=12 kHz (fN=6 kHz):

- 7 kHz: |​7−12|​=5 kHz⟹7 kHz→5 kHz

- 9 kHz: |​9−12|​=3 kHz⟹9 kHz→3 kHz

- 10 kHz: |​10−12|​=2 kHz⟹10 kHz→2 kHz

- 13 kHz: |​13−12|​=1 kHz⟹13 kHz→1 kHz

## 16. Visualizing Spectral Folding

At fs=12 kHz, frequencies reflect symmetrically around the Nyquist boundary fN=6 kHz:

| Input f (kHz) | Folded falias (kHz) |
| --- | --- |
| 0 | 0 |
| 2 | 2 |
| 6 | 6 |
| 7 | 5 |
| 10 | 2 |
| 12 | 0 |

## 17. Normalized Frequency Perspective

Re-evaluating the 10 kHz input at fs=12 kHz in normalized frequency:

ω=2π1012=5π3

Modular reduction yields:

5π3≡−π3 (mod 2π)

Taking the real part for a cosine:

cos−π3n=cosπ3n

Converting back to linear frequency in Hz:

f=π/32π⋅12000=2000 Hz

## 18. Pre-ADC Filtering Requirements

If an analog input contains a desired 2 kHz component alongside an unwanted 10 kHz component, sampling at fs=12 kHz maps both to 2 kHz:

xn=A1cos2π2000fsn+A2cos2π2000fsn=A1+A2cos2π2000fsn

Because the terms combine linearly into a single 2 kHz discrete component, no digital filter can separate them post-sampling. The anti-aliasing filter must exist in the analog domain prior to sampling:

Sensor→Analog Anti-Alias Filter→ADC→Digital Processing

## 19. Digital Inability to Fix Aliasing

Digital post-processing cannot fix aliasing because violation of the Nyquist criterion represents a many-to-one mapping. Overlapping spectral content destroys linear independence between baseband and alias frequencies.

## 20. Downsampling / Decimation

Downsampling an already digitized signal requires pre-filtering. Converting an audio stream from 48 kHz to 12 kHz reduces the Nyquist frequency from 24 kHz to 6 kHz.

Low-pass filter first, decimate second.

- Direct slicing (x[::4]) without filtering causes high frequencies (6–24 kHz) to alias into the 0–6 kHz band.

- Polyphase filtering (scipy.signal.resample_poly) suppresses content above 6 kHz prior to decimation.

## 21. Mechanism of Naive Decimation Aliasing

Consider a 10 kHz component in a 48 kHz signal (fN=24 kHz). Selecting every 4th sample drops fs to 12 kHz (fN′=6 kHz). The 10 kHz component now exceeds fN′ and folds:

|​10−12|​=2 kHz⟹10 kHz→2 kHz

## 22. Proper Decimation Trade-offs

Proper decimation strips energy above the target Nyquist rate (6 kHz) using a digital low-pass filter before sample removal. Information above 6 kHz is lost by design, avoiding false low-frequency artifacts.

## 23. Downstream Machine Learning Vulnerabilities

If audio containing an un-filtered 10 kHz tone undergoes naive decimation, a 2 kHz line appears in the resulting spectrogram.

Strong energy at 2 kHz

A convolutional neural network (CNN) trained on these features will treat this line as valid 2 kHz acoustic data, potentially causing the model to learn hardware-dependent sampling artifacts rather than true physical class features.

## 24. Dataset Shortcuts

In machine learning, if Class A recordings originate from an ADC with poor anti-alias filtering while Class B recordings do not, a CNN may achieve 100% training accuracy strictly by classifying hardware artifacts. When deployed on standard hardware, performance collapses.

## 25. Application Problem 1

Scenario: A sensor signal is bandlimited to 18 kHz. Is fs=32 kHz sufficient?

- Calculation: Nyquist Rate=2fM=218=36 kHz.

- Evaluation: 32 kHz<36 kHz.

- Verdict: No. The replicas will overlap because fN=16 kHz<18 kHz.

## 26. Application Problem 2

Scenario: A 17 kHz tone is sampled at fs=20 kHz. What frequency appears in the output?

- Calculation: fN=10 kHz.

- Alias Calculation: |​17−20|​=3 kHz.

- Normalized Frequency Verification:

ω=2π1720=1.7π≡−0.3π (mod 2π)

f=0.3π2π20 kHz=3 kHz

## 27. Application Problem 3

Scenario: Why can't a digital low-pass filter placed after the ADC remove aliasing?

Once sampled, the aliased component and legitimate in-band signal at the same frequency merge into a single sequence value. Information distinguishing their origins is destroyed.

## 28. Ultrasonic Interference

Even if useful acoustic signals top out at 20 kHz and fs=48 kHz (fN=24 kHz), out-of-band ultrasonic noise, radio frequency demodulation, or circuit harmonics above 24 kHz can still alias into the audible spectrum without an analog anti-aliasing filter.

## 29. Practical Bandlimiting

Theoretical signals satisfy XcjΩ=0 for |​Ω|​>ΩM. Real physical signals are rarely strictly bandlimited, requiring analog filtering to attenuate out-of-band energy below the dynamic range floor of the ADC.

## 30. Band Support Boundaries

Consider a continuous signal with spectral support in the range −4 kHz,+4 kHz.

- Case A (fs=12 kHz): Replicas center at 0,±12 kHz.

- Baseband: −4,+4 kHz

- First replica: 8,16 kHz

- Result: No spectral overlap.

- Case B (fs=6 kHz): Replicas center at 0,±6 kHz.

- Baseband: −4,+4 kHz

- First replica: 2,10 kHz

- Result: Overlap occurs in the 2,4 kHz region.

## 31. Single Sinusoid Ambiguity

Aliasing is irreversible from the samples alone.

Without external prior assumptions regarding signal bandlimits, an infinite set of continuous-time frequencies {|​falias±kfs|​} maps to the exact same sample sequence.

## 32. Physical ADC Architecture

A complete continuous-to-digital hardware chain contains distinct functional blocks:

Analog Input→Anti-Alias Filter→Sample-and-Hold→Quantizer→Digital Code

## 33. Aliasing vs. Quantization Noise

- Aliasing: Caused by under-sampling in time (fs<2fM). Rotates high-frequency energy into false lower frequencies.

Aliasing changes frequency identity.

- Quantization: Caused by finite bit-depth resolution in amplitude. Modeled as additive noise en=xqn−xn.

Quantization introduces amplitude error.

## 34. Checkpoint Questions

- A 5 kHz sinusoid is sampled at 8 kHz. What frequency appears digitally?

- Calculation: |​5−8|​=3 kHz.

- Answer: 3 kHz.

- A signal is bandlimited to 6 kHz. Which rate satisfies Nyquist with margin?

- Calculation: 2fM=12 kHz. Rate must strictly exceed 12 kHz.

- Answer: 16 kHz.

- Which component must appear first in an ADC front-end?

- Answer: Analog anti-aliasing filter.

- Why is aliasing problematic for ML models?

- Answer: It introduces false, structured spectral features into the input data.

- At fs=16 kHz, which two frequencies produce identical discrete cosines?

- Calculation: |​16−13|​=3 kHz.

- Answer: 3 kHz and 13 kHz.

## 35. Core Concepts

- Spectral Replication Origin: Time-domain multiplication with an impulse train corresponds to frequency-domain convolution with an impulse train.

- Factor of 2 in Nyquist: A real bandlimited spectrum extending from −fM to +fM occupies a bandwidth of 2fM.

- Anti-Alias Filtering Location: Must occur in the analog domain to remove energy above fN before sampling makes it indistinguishable from lower-frequency content.

## 36. Information-Theoretic Summary

No overlap⇒unique reconstruction;  overlap⇒aliasing and non-uniqueness

If fs>2fM, spectral replicas remain isolated, guaranteeing unique signal recovery. If fs<2fM, spectral overlap destroys uniqueness.

## 37. Final Conceptual Mapping

xct→sample every Txn

xct×n​δt−nT

XsjΩ=1Tk=−∞∞XcjΩ−kΩs

Spectral replicas

fs>2fM⇒Replicas separate

fs<2fM⇒Replicas overlap⇒Aliasing

Once false structure enters the samples, downstream models treat it as valid data.

That is the larger reason this lecture belongs early in a DSP + ML course: students learn that the data acquisition chain is part of the machine-learning system, not merely something that happened before the “interesting” ML began.

# Week 2, Lecture 4, LaTeX math

Here is the lecture content converted with properly formatted LaTeX for all the mathematical expressions, organized for clear readability:

This lecture covers signal reconstruction, continuous-to-discrete amplitude mapping, and the theoretical and practical foundations of quantization.

When do samples faithfully represent the original analog signal?

Lecture 4 addresses two key follow-up questions:

If the samples are valid, how do we turn them back into a continuous waveform?

What happens when amplitudes can only be represented with a finite number of bits?

This presents two distinct analytical paths:

samples→ideal reconstruction→practical DAC/ZOH

continuous amplitude→quantization→error/noise→SQNR

Sampling error (aliasing) shifts energy to incorrect frequencies and creates non-uniqueness, whereas quantization maps continuous amplitudes onto a finite set of discrete levels.

### 1. Distinguishing the Two Kinds of Discretization

- Sampling Discretizes Time: Starting with continuous-time signal xct, sampling yields discrete-time sequence:

xn=xcnT

In an ideal mathematical model, amplitudes xn remain unconstrained continuous real values.

- Quantization Discretizes Amplitude: Quantization maps continuous amplitude values to discrete levels.

sampling: continuous time→discrete time

quantization: continuous amplitude→discrete amplitude levels

### 2. Ideal Reconstruction: Signal Recovery

Ideal sampling creates spectral replicas:

XsjΩ=1Tk=−∞∞XcjΩ−kΩs

When the Nyquist criterion is satisfied, adjacent copies do not overlap. Reconstructing the original signal requires isolating the baseband spectrum centered at Ω=0 while rejecting all other copies using an ideal reconstruction low-pass filter:

HrjΩ=T,|​Ω|​<Ωc0,otherwise

### 3. Reconstruction Filter Gain T

Inside the baseband region, the sampled spectrum is:

XsjΩ=1TXcjΩ

Applying a unit-gain filter leaves an amplitude scaling of 1TXcjΩ. Setting filter gain HrjΩ=T in the passband balances the scale factor:

T1TXcjΩ=XcjΩ

The filter does not construct missing signal information; under bandlimited conditions, the samples contain sufficient information to uniquely specify the continuous waveform.

### 4. Mathematical Origin of Sinc Interpolation

An ideal rectangular brick-wall filter in the frequency domain corresponds to a sinc function in the time domain:

hrt=sinctT

Convolving the impulse train with hrt yields the ideal continuous-time reconstruction formula:

xrt=n=−∞∞xnsinct−nTT

where normalized sinc is defined as:

sincu=sinπuπu

Each sample weight scales a centered, shifted sinc kernel.

### 5. Physical Meaning of Sinc Interpolation

Every sample xn scales a sinc function centered at t=nT. The reconstructed signal is the infinite sum of all shifted kernels across time:

xrt=⋯+x−1sinct+TT+x0sinctT+x1sinct−TT+⋯

Because the sinc function has infinite support, every sample influences intermediate signal values across all time.

### 6. Sample Point Exact Intersections

The normalized sinc function satisfies:

sinc0=1 and sinck=0 ∀k∈Z,k≠0

Evaluating reconstruction at sample instant t=mT:

xrmT=n=−∞∞xnsincm−n

All terms evaluate to zero except where n=m, yielding:

xrmT=xm

At t=3T, the contribution from the sample at n=1 uses argument 3−1=2:

sinc2=0⟹0

### 7. Uniqueness Between Sample Points

For arbitrary unconstrained functions, infinitely many curves pass through a set of discrete points. Under the assumption that the original signal is strictly bandlimited below fs2, only one unique continuous signal can generate those exact sample values.

### 8. Physical DAC Constraints

Ideal sinc interpolation cannot be implemented directly in physical hardware due to:

- Infinite time duration (infinite support)

- Non-causality (hrt≠0 for t<0)

- Unachievable ideal rectangular frequency response

- Incomplete bandlimiting in physical inputs

Hardware uses causal continuous-time approximations, such as the Zero-Order Hold (ZOH).

### 9. Zero-Order Hold (ZOH) Functionality

A ZOH converter maintains a constant voltage output equal to the most recent sample value until the subsequent clock edge:

1.0  ──────────┐
               │
0.5            └──────────┐
                          │
0                    -----│
                          └────────
-0.2
       0          T          2T

The resulting piecewise-constant staircase is a continuous-time analog signal containing high-frequency spectral images and low-pass magnitude droop.

### 10. ZOH as Pulse Convolution

Impulse-train representation:

xst=n=−∞∞xnδt−nT

Defining the rectangular pulse pt:

pt=1,0≤t<T0,otherwise

The ZOH output equals the convolution of impulses with the pulse shape:

xZOHt=xst*pt

impulse*rectangle=shifted rectangle

### 11. ZOH Frequency Response Derivation

Taking the Fourier transform of pulse pt=ut−ut−T:

HZOHjΩ=0Te−jΩt dt=1−e−jΩTjΩ

Factoring symmetrically:

1−e−jΩT=e−jΩT/2ejΩT/2−e−jΩT/2=2je−jΩT/2sinΩT2

Substituting into the transfer function:

HZOHjΩ=Te−jΩT/2sinΩT/2ΩT/2

HZOHjΩ=Te−jΩT/2sincΩT2π

This response consists of a linear-phase delay term combined with a sinc magnitude envelope.

### 12. Linear-Phase Delay Factor

The exponential factor e−jΩT/2 represents a fixed time delay of half the sample period:

t0=T2

This delay arises because the holding pulse extends forward in time from t=0 to t=T rather than being centered at t=0.

### 13. ZOH Magnitude Response & Attenuation

Ignoring constant scale factor T:

​HZOHjΩT​=​sinΩT/2ΩT/2​

- Near DC (Ω≈0): sinxx≈1

- Higher frequencies suffer increasing attenuation up to the first spectral zero at Ω=2πT

ZOH attenuates high frequencies more than low frequencies.

### 14. Quantifying Nyquist Droop

At the digital Nyquist limit (ΩT=π):

|​H|​T=sinπ/2π/2=2π≈0.6366

Expressed in decibels:

20log100.6366≈−3.92 dB

This −3.92 dB attenuation near Nyquist is deterministic frequency shaping inherent to holding continuous samples, not random noise.

### 15. Role of the Analog Reconstruction Filter

The ZOH envelope attenuates high-frequency spectral replicas centered at fs,2fs,… but does not eliminate them entirely:

xn→DAC/ZOH→Analog Reconstruction LPF→xrt

The post-DAC analog low-pass filter suppresses remaining high-frequency images.

### 16. Oversampling Advantage

- Base Rate Sampling (fs=44.1 kHz): Passband ends at 20 kHz and Nyquist is 22.05 kHz, requiring steep analog filter roll-off in a 2.05 kHz transition band.

- Oversampled Conversion (fs=176.4 kHz): First spectral images are pushed up to 176.4 kHz, allowing a gradual analog filter transition band.

### 17. Quantization Dynamics

Quantization converts continuous amplitude values into B-bit numerical codes:

L=2B

| Resolution (B) | Code Levels (L=2B) |
| --- | --- |
| 4 bits | 16 |
| 8 bits | 256 |
| 16 bits | 65,536 |
| 24 bits | 16,777,216 |

Inputs exceeding the input voltage range [−A,A) cause saturation clipping.

### 18. Quantizer Step Size Formula

For dynamic range [−A,A) with total width 2A split across 2B levels:

Δ≈2A2B

- For A=1,B=8: Δ≈2256=0.0078125

- For A=1,B=4: Δ≈216=0.125

### 19. Additive Quantization Error Model

Quantization error en is defined as:

en=xqn−xn⟹xqn=xn+en

For round-to-nearest quantizers, error bounds are:

−Δ2≤en<Δ2

### 20. Statistical Noise Model Assumptions

Quantization is deterministic, but error sequence en can be modeled as zero-mean uniform white noise:

en∼Uniform−Δ2,Δ2

This model applies when signal amplitude spans many quantization steps, possesses complex phase structure, and quantization resolution is sufficiently high.

### 21. Derivation of Error Variance σe2

For uniformly distributed random variable e∼U−Δ2,Δ2 with pe=1Δ:

σe2=Ee2=−Δ/2Δ/2e21Δ de

σe2=1Δe33−Δ/2Δ/2=1ΔΔ324−−Δ324

σe2=Δ212

### 22. Derivation of 6 dB/Bit Rule

Incrementing bit depth by 1 bit doubles available levels:

2B→2B+1⟹Δ→Δ2

Error power scales quadratically with step size:

σe2∝Δ2⟹σe,new2=Δ/2212=14σe2

In decibels:

10log104≈6.02 dB

one additional bit≈6.02 dB improvement

### 23. Full-Scale Sinusoid SQNR Derivation

For full-scale sinusoid xn=Asinωn:

- RMS Amplitude: A2

- Signal Power: Px=A22

- Step Size: Δ=2A2B

- Noise Power: Pe=Δ212=1124A222B=A23⋅22B

Calculating Signal-to-Quantization-Noise Ratio:

SQNR=PxPe=A2/2A2/3⋅22B=32⋅22B

Converting to decibels:

SQNRdB=10log1032+20Blog102

SQNR≈6.02B+1.76 dB

### 24. Standard SQNR Reference Values

- 8-bit: 6.028+1.76=49.92 dB≈50 dB

- 12-bit: 6.0212+1.76=74.00 dB≈74 dB

- 16-bit: 6.0216+1.76=98.08 dB≈98.1 dB

### 25. 96 dB vs. 98.1 dB Dynamic Range Distinction

- 6.02×16=96.32 dB estimates raw 16-bit dynamic range.

- 6.0216+1.76=98.08 dB calculates ideal SQNR specifically for a full-scale sinusoid.

### 26. Signal Power Dependencies

The 1.76 dB constant assumes a full-scale sinusoid. If signal amplitude drops to 0.1A (−20 dB input level) while step size Δ remains fixed, actual SQNR drops by 20 dB.

### 27. Quantization vs. Overload Clipping

- Quantization Error: Bounded round-off within [−Δ2,Δ2).

- Clipping: Non-linear waveform truncation occurring when |​xt|​>A. Creates severe harmonic distortion.

### 28. Coarse Quantization Artifacts (4-bit Audio)

At low resolutions (B=4, 16 levels), error terms correlate strongly with signal trajectories, generating coarse, harmonic distortion rather than white additive noise.

### 29. Low-Amplitude Periodic Error

Low-amplitude sinusoids crossing only 2–3 quantization levels yield periodic error signals. Periodic errors introduce distinct harmonic spectral lines instead of uniform noise floors.

### 30. Function of Dither

Dither adds small-amplitude random noise (e.g., triangular probability density function with amplitude ±Δ) prior to quantization. Dither decorrelates quantization error from the input signal, converting structured harmonic distortion into stationary, non-objectionable white noise.

structured distortion→randomized noise floor

### 31. Machine Learning Sensitivity to Artifacts

Models trained on coarsely quantized inputs without dither may learn false, structured amplitude artifacts as valid class features.

### 32. Numerical Type Casting (int16 to float32)

Converting 16-bit PCM integer samples to floating-point tensors changes computational precision, not underlying acquisition measurement resolution.

x = pcm.astype(np.float32) / 32768.0  # Scales numeric range to [-1.0, 1.0)

### 33. Information Invariance Under Type Conversion

Casting an int16 value to float32 does not recover amplitude detail removed during quantization.

more bits in container≠more measurement detail

### 34. Non-Reversibility of Hardware Distortion

Type casting to float64 cannot restore information lost to aliasing or quantization.

### 35. Precision Conceptual Check

Loading 16-bit audio into PyTorch as a torch.float32 tensor updates the arithmetic container format used for gradient updates and operations, leaving physical measurement resolution fixed at 16 bits.

### 36. Audio Demonstration Progression

- Clean 48 kHz: Baseline reference.

- Naive 48 kHz → 12 kHz: Severe aliasing fold-over (false frequencies).

- Filtered 48 kHz → 12 kHz: Bandwidth reduced above 6 kHz without aliasing distortion.

- 8-bit Quantization: Higher noise floor, speech remains intelligible.

- 4-bit Quantization: Heavy signal-correlated granular distortion.

### 37. Qualitative Distinction: Aliasing vs. Quantization

- Aliasing: Maps spectral energy to incorrect baseband locations, causing irreversible frequency overlap.

- Quantization: Perturbs sample amplitude values, adding noise or harmonic distortion.

### 38. Structural Comparison: ZOH Droop vs. Quantization Noise

- ZOH Droop: Deterministic continuous-time linear magnitude attenuation (sinc shaped).

- Quantization Noise: Amplitude error statistical approximation (Δ212 variance).

### 39. Core Conceptual Answers

- Ideal Reconstruction Sinc Basis: The ideal low-pass reconstruction filter has a rectangular spectrum in frequency, corresponding to a sinct impulse response in time.

- ZOH Frequency Impact: Introduces a sinc-shaped attenuation envelope (−3.92 dB at Nyquist) and a T/2 linear-phase delay.

- 12-bit Full-Scale Sine SQNR:

SQNR=6.0212+1.76=74.0 dB

### 40. Verification Questions

- Sinc Reconstruction Contribution: Each sample weight launches an infinitely long shifted sinc function.

- Zero-Order Hold Core Function: Holds sample amplitudes constant across sampling interval T.

- Adding One Bit: Step size Δ halves, noise power drops by a factor of 4, improving SQNR by ≈6 dB.

- int16 to float32 Conversion: Changes internal compute precision without increasing acquisition information.

- Unmodeled Behavior: Overload clipping falls outside the standard Δ212 uniform noise model.

### 41. Level-Based Analysis Summary

- Undergraduate/Senior Formulations:

Δ≈2A2B, σe2=Δ212, SQNR≈6.02B+1.76 dB

- Graduate Formulations: Integration of e2pe, bounds of uniform noise models, signal-error correlation dynamics, decorrelation via dither, and ZOH Fourier transform derivations.

### 42. End-to-End Acquisition & Reconstruction Chain

- Anti-Alias Filter: xct→Analog LPF (suppresses frequencies above fN).

- Sampling: xn=xcnT (discretizes time).

- Quantization: xqn=xn+en (discretizes amplitude).

- Type Casting: int16 → float32 (prepares storage container for computation).

- D/A Conversion: ZOH produces staircase waveform xZOHt=xst*pt.

- Reconstruction Filter: Suppresses spectral images around kfs.

### 43. Hardware-Software Pipeline Principles

Sampling determines whether frequency information survives.

Quantization determines how accurately amplitude survives.

Reconstruction determines how digital values map back to physical waveforms.

That gives students a much stronger conceptual basis than treating “ADC,” “DAC,” “bit depth,” and “sample rate” as isolated engineering vocabulary.

## Week 2, Lecture 3, Exit Tickets

What are good exit ticket questions with answers for this lecture?

Chat GPT (Medium):

For this lecture, I’d make the exit ticket short enough to finish in about 3–5 minutes, but design it to test whether students understand the reasoning behind sampling rather than just remembering (f_s>2f_M). Your lecture emphasizes the sampling model, spectral replication, aliasing as non-uniqueness, and why anti-alias filtering must occur before sampling or decimation.

Here are some strong choices.

- A continuous-time signal contains frequencies only up to 7 kHz. What is the minimum theoretical sampling rate required to avoid aliasing?

- Answer:

fs>2fM=14 kHz

- So theoretically the sampling rate must be greater than 14 kHz. In practice, you would choose something higher to allow room for the anti-alias filter transition band.

- What this tests: Whether students distinguish the signal bandwidth from the Nyquist frequency and can apply the no-overlap condition. Your slides explicitly distinguish the Nyquist frequency (f_s/2) from the Nyquist rate (2f_M).

- A 9 kHz sinusoid is sampled at (f_s=12) kHz. What frequency will appear in the sampled data?

- Answer: The Nyquist frequency is 6 kHz. The 9 kHz sinusoid folds around 6 kHz:

falias=9−12=3 kHz.

- So the sampled data looks like a 3 kHz sinusoid.

- What this tests: Whether students can actually predict an alias rather than only say “aliasing occurs.”

- True or false: If aliasing occurs during sampling, we can usually remove it afterward with a digital low-pass filter. Explain in one sentence.

- Answer: False. Once aliasing occurs, an aliased component and a genuine in-band component can produce the same samples, so the digital system cannot determine which one was originally present.

- This is one of the most important conceptual points in the lecture: aliasing is a loss of uniqueness, not simply extra noise.

- Why does sampling create repeated copies of the analog spectrum?

- Good answer: Because ideal sampling multiplies the continuous-time signal by a periodic impulse train. Multiplication in time corresponds to convolution in frequency, and the Fourier transform of the impulse train is itself an impulse train. Convolving (X_c(j\Omega)) with those impulses creates shifted copies of the spectrum spaced by the sampling frequency.

- Excellent student answer: “Sampling multiplies (x_c(t)) by an impulse train; in frequency this becomes convolution with another impulse train, producing replicas of (X_c(j\Omega)) every (\Omega_s).”

- That is almost exactly the central derivation developed in the lecture.

- Suppose two different continuous-time sinusoids produce exactly the same discrete-time samples. What phenomenon does this illustrate?

- Answer: Aliasing.

- Better answer: Aliasing demonstrates that the mapping from continuous-time signals to samples is no longer one-to-one; multiple analog signals are consistent with the same digital sequence.

- What this tests: Whether they understand the deeper idea that aliasing is non-uniqueness rather than merely “frequencies getting messed up.” Your lecture makes that distinction explicitly.

- You have audio sampled at 48 kHz and want to reduce the rate to 12 kHz. Why is x[::4] potentially wrong? What should you do instead?

- Answer: Keeping every fourth sample changes the Nyquist frequency from 24 kHz to 6 kHz. Any energy above 6 kHz can therefore alias into the new baseband.

- Correct procedure:

Low-pass filter first→downsample by 4

- This connects sampling theory directly to the practical decimation example in the lecture.

- A 10 kHz component is present in a 48 kHz recording. The recording is naively downsampled to 12 kHz. Where will the 10 kHz component appear?

- Answer:

falias=10−12=2 kHz.

- It appears falsely as a 2 kHz component.

- Follow-up: Why could this be particularly dangerous for a machine-learning classifier?

- Answer: Because the classifier may learn the repeatable 2 kHz alias as though it were a real physical feature of the signal. Your lecture specifically emphasizes that a CNN cannot know that this structure originated as an alias.

- Complete the sentence: An anti-alias filter must be placed ______ the ADC because ______.

- Answer: “An anti-alias filter must be placed before the ADC because once out-of-band energy folds into the sampled baseband, its original frequency cannot generally be recovered.”

- What this tests: Whether students understand the physical ordering, not merely the vocabulary.

For this particular lecture, my favorite three-question exit ticket would be:

1. A 9 kHz tone is sampled at 12 kHz. What frequency appears in the samples? Answer: 3 kHz.

2. Why can't a digital filter generally remove aliasing after sampling? Answer: Because after frequencies fold together, genuine in-band and aliased components are indistinguishable.

3. In one or two sentences, explain why sampling creates spectral replicas. Answer: Sampling multiplies the signal by a periodic impulse train. Multiplication in time becomes convolution in frequency, creating shifted copies of the original spectrum at multiples of the sampling frequency.

That combination is particularly useful because Question 1 tests calculation, Question 2 tests physical/conceptual understanding, and Question 3 tests whether they understood the mathematical mechanism. Together they give you a better diagnosis than three Nyquist-rate calculations.

One question I would not use as the primary exit-ticket question is simply, “State the Nyquist sampling theorem.” A student can memorize (f_s>2f_M) and still misunderstand essentially everything important in this lecture. Your stated learning objective is specifically that students should be able to derive—not just quote—the sampling theorem, so the exit ticket should reflect that goal.

## Week 2, Lecture 4, Exit Tickets

What are good exit ticket questions with answers for this lecture?

Chat GPT (Medium):

For this lecture, I’d make the exit ticket test three different things: ideal reconstruction, practical DAC behavior, and quantization/SQNR. That matches the lecture’s stated outcomes very well.

Here are several good choices with answers.

- Why does ideal reconstruction use sinc functions?

- Answer: Because the ideal reconstruction filter is a brick-wall low-pass filter, and its impulse response is a sinc function. Therefore, reconstruction is performed by summing shifted sinc functions weighted by the sample values.

- A strong student answer would be: “Each sample scales a shifted sinc, and the sinc functions interpolate exactly through the original sample values.”

- This directly follows the lecture’s derivation from ideal low-pass filtering to sinc interpolation.

- Why does sinc interpolation pass exactly through the sample values?

- Answer: Because the normalized sinc function is zero at every nonzero integer offset and equals 1 at zero. At (t=mT), all sinc terms vanish except the one centered at (mT).

- So:

xrmT=xm.

- This is a very good conceptual check because it shows whether students understand why sinc interpolation actually works rather than simply recognizing the formula.

- Why don't practical DACs usually implement ideal sinc interpolation directly?

- Answer: Because an ideal sinc impulse response has infinite duration and is noncausal. A real DAC instead usually produces a simpler waveform such as a zero-order hold and then uses an analog reconstruction filter.

- The lecture explicitly contrasts ideal interpolation with the practical DAC approach.

- What does a zero-order hold do in the time domain?

- Answer: It holds each sample value constant for one sampling period.

- Follow-up: What does that do in frequency?

- Answer: It introduces a sinc-shaped magnitude response, causing progressively greater attenuation at higher frequencies.

- This makes a nice two-part exit-ticket question because students must connect the time-domain staircase to the frequency-domain droop.

- True or false: The frequency droop produced by a zero-order hold is random noise.

- Answer: False. It is a deterministic frequency-response effect.

- Better answer: The ZOH has a sinc-shaped transfer-function magnitude, so higher-frequency components are attenuated in a predictable way.

- That distinction is explicitly emphasized in the lecture.

- A DAC uses a zero-order hold. What is the purpose of the analog reconstruction filter that follows it?

- Answer: Primarily to remove spectral images around multiples of the sampling frequency. It can also help compensate some of the ZOH passband droop.

- This tests whether students see the DAC and analog filter as one reconstruction chain rather than treating the DAC output as the final waveform.

- What is the difference between sampling and quantization?

- Answer:

  - Sampling discretizes time.

  - Quantization discretizes amplitude.

- This is simple, but it is worth asking because students often mix the two concepts. The lecture makes this distinction explicitly.

- How many nominal quantization levels does an (8)-bit quantizer have?

- Answer:

28=256.

- Follow-up: What happens to the quantization step Δ if you add one more bit?

- Answer: It is approximately cut in half.

- Since quantization-noise power is proportional to Δ2, halving Δ reduces the noise power by approximately a factor of 4.

- Approximately how much does ideal SQNR improve when one bit is added?

- Answer: About

6 dB per bit.

- More precisely, for a full-scale sine,

SQNR≈6.02B+1.76 dB.

- The “6 dB per bit” result comes from the fact that one extra bit halves Δ, reducing quantization-noise power by a factor of 4.

- Estimate the ideal SQNR of a full-scale 10-bit sinusoid.

Answer:

SQNR≈6.0210+1.76

≈61.96 dB

or about 62 dB.

- Estimate the ideal SQNR of a full-scale 12-bit sinusoid.

Answer:

6.0212+1.76=74.0 dB.

This is also the calculation already included in the lecture's existing exit check.

- Why is the common statement “16-bit audio has about 96 dB of dynamic range” slightly different from the (98.1) dB SQNR result?

Answer: They use slightly different reference calculations. Roughly (6) dB/bit gives about (96) dB, while

6.02B+1.76

gives approximately (98.1) dB for a full-scale sine because it incorporates the RMS power of the sine and the uniform quantization-error model.

That is a stronger exit question for an advanced class because it tests whether students understand where the numbers come from rather than treating them as conflicting facts.

- A recording is stored as 16-bit PCM. You convert it to a PyTorch float32 tensor. Has the signal now gained additional amplitude resolution?

Answer: No.

The representation used for subsequent computation has changed, but the information captured during quantization has not. Values between the original 16-bit quantization levels cannot be recovered just by casting the data to float32.

This would be one of my favorite exit-ticket questions because it connects the DSP material directly to the later ML part of the course.

- True or false: If an audio recording is stored as float32, it cannot contain significant quantization error.

Answer: False.

A float32 tensor may simply contain values that originated from a much lower-resolution ADC or PCM file. More computational precision does not restore information that was lost during acquisition.

- The additive quantization-noise model assumes quantization error is approximately uniform and uncorrelated with the signal. Is this always true?

Answer: No.

It is a useful approximation when the signal spans many quantization levels and the error is sufficiently decorrelated from the signal. Low-level signals can produce structured or signal-correlated quantization error.

This is a very good higher-level conceptual question because it tests whether students understand the distinction between a mathematical model and physical reality.

For a 3-question exit ticket, I would probably use these:

1. Explain in one sentence why ideal reconstruction produces sinc interpolation.

Answer: Because the ideal reconstruction system is a brick-wall low-pass filter whose impulse response is a sinc.

2. A DAC uses a zero-order hold. What happens to its frequency response as frequency increases?

Answer: The response follows a sinc-shaped envelope, so higher frequencies experience progressively greater magnitude droop.

3. An audio file recorded with a 12-bit ADC is converted to float32 for PyTorch. Does it now have 32-bit measurement accuracy? Explain.

Answer: No. float32 gives more numerical precision for computation, but it cannot recreate amplitude information lost during the original 12-bit quantization.

I particularly like that set because the questions move from ideal theory → practical hardware → DSP/ML interpretation, which is exactly the progression of the lecture. The lecture summary itself follows essentially that same structure: sinc interpolation, ZOH behavior, quantization/SQNR, and the distinction between PCM bit depth and computational dtype.

If you want a slightly more quantitative exit ticket, I would substitute Question 3 with:

A full-scale 8-bit sinusoid has approximately what ideal SQNR?

6.028+1.76≈49.9 dB.

That one is useful because a student who answers 48 dB is showing that they remember the (6\text{ dB/bit}) approximation, while a student who gets about 50 dB understands the full-scale-sine formula. Both responses tell you something useful about where they are conceptually.